5.3	Positioning accuracy enhancements

Finalization of representative sub-use cases:

The following are selected as representative sub-use cases:

-	Direct AI/ML positioning:

-	AI/ML model output: UE location

-	e.g., fingerprinting based on channel observation as the input of AI/ML model

-	AI/ML assisted positioning:

-	AI/ML model output: new measurement and/or enhancement of existing measurement

-	e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement

More specifically, the following Cases are considered for the study:

-	Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

-	Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

-	Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

-	Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

-	Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

One-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

For all five positioning cases (Case 1/2a/2b/3a/3b), RAN1 has not considered prioritization.

For positioning enhancement use case:

-	For model training, training data can be generated by UE/PRU/gNB/LMF.

-	For LMF-side model inference (Case 2b, Case 3b), input data can be generated by UE/gNB and terminated at LMF.

-	For gNB-side model inference (Case 3a), input data is internally available at gNB.

-	For UE-side model inference (Case 1, Case 2a), input data is internally available at UE.

-	For performance monitoring at the LMF side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by UE/gNB and terminated at LMF.

-	For performance monitoring at the gNB side, calculated performance metrics (if needed) or data needed for performance metric calculation (if needed) can be generated by at least gNB.

代表性子用例的确定：

以下被选为代表性子用例：

直接AI/ML定位：

AI/ML模型输出：终端（UE）位置

例如：基于信道观测作为AI/ML模型输入的指纹定位

AI/ML辅助定位：

AI/ML模型输出：新的测量量和/或对现有测量量的增强

例如：视距/非视距（LOS/NLOS）识别、定时和/或角度测量、测量似然度

更具体地说，本次研究考虑以下用例：

用例1：基于UE的定位，采用UE侧模型，直接AI/ML或AI/ML辅助定位

用例2a：UE辅助/基于LMF的定位，采用UE侧模型，AI/ML辅助定位

用例2b：UE辅助/基于LMF的定位，采用LMF侧模型，直接AI/ML定位

用例3a：NG-RAN节点辅助定位，采用gNB侧模型，AI/ML辅助定位

用例3b：NG-RAN节点辅助定位，采用LMF侧模型，直接AI/ML定位

在Rel-18 SI中，优先考虑推理完全在终端（UE）或完全在网络侧执行的单侧模型。

对于上述所有五个定位用例（用例1/2a/2b/3a/3b），RAN1尚未考虑优先级排序。

针对定位增强用例：

模型训练：训练数据可由UE/定位参考单元（PRU）/gNB/定位管理功能（LMF）生成。

LMF侧模型推理（用例2b、用例3b）：输入数据可由UE/gNB生成，并在LMF终结。

gNB侧模型推理（用例3a）：输入数据在gNB内部可用。

UE侧模型推理（用例1、用例2a）：输入数据在UE内部可用。

LMF侧的性能监控：计算得到的性能指标（如果需要）或性能指标计算所需的数据（如果需要）可由UE/gNB生成，并在LMF终结。

gNB侧的性能监控：计算得到的性能指标（如果需要）或性能指标计算所需的数据（如果需要）可至少由gNB生成