3GPP TSG-RAN WG1 Meeting #120	Tdoc R1-2500060

Athens, Greece, February  17th – 21nd, 2025

Agenda Item:	9.1.2

Source:	Ericsson

Title:	AI/ML for Positioning Accuracy Enhancement

Document for:	Discussion, Decision

1	Introduction	2

2	Integrating AI/ML based positioning methods with existing protocol	3

2.1	Cases with AI/ML model hosted in the UE (Case 1)	4

2.2	Case with AI/ML model hosted in the TRP/gNB (Case 3a)	4

2.3	Case with AI/ML model hosted in the LMF (case 3b)	5

3	Signaling enhancement for model inputs	6

3.1	Remaining issues for sample-based measurements for model input	7

3.1.1	Value range for parameters {Nt, Nt', k}	9

3.1.2	LMF handling of sample-based or path-based reporting	12

3.1.3	Total-power PDP report for Rel-19	13

3.2	Analysis of supporting phase information measurement	15

3.2.1	Inapplicability of Rel-18 carrier phase positioning	16

3.2.2	Issues with random initial phase	16

3.2.3	Difficulty to align phase measurements between model training and model inference	20

3.2.4	Signaling size vs positioning accuracy tradeoff analysis	22

3.2.4.1	Simple signaling size comparison based on the number of real numbers to signal	23

3.2.4.2	Signaling sizes based on simple direct sample timing bitmaps	27

3.3	Impact of AI/ML model location for Case 3a	29

4	Model output of AI/ML assisted positioning	30

4.1	Details for reporting model output of AI/ML assisted positioning	30

4.1.1	LOS/NLOS indicator	30

4.1.1.1	Interpretation of LOS/NLOS indicator and associated timing information	31

4.1.1.2	Reporting of a of LOS/NLOS indicator based on model output	34

4.1.1.3	Summary of LOS/NLOS indicator reporting	34

4.1.2	Timing information	36

4.1.3	Indication of AI/ML assistance in generating measurement reports	38

5	Training data collection	38

5.1	Content of training data samples	38

5.1.1	Time stamp of training data samples	39

5.1.2	Signalling and quality reporting for Part A	43

5.1.3	Signalling and quality reporting for Part B	44

5.1.4	Pairing of Part A and Part B in data collection	49

5.2	Metadata for training dataset	50

5.2.1	Elements in metadata of the training dataset	50

5.2.2	Assistance data for consistency between training and inference	53

6	LCM and Model performance monitoring	56

6.1	Model-level and functionality-level monitoring	56

6.2	Self-monitoring: Label-free or label-based	60

6.3	Model performance monitoring metric calculation for Case 1	63

7	Model inference	65

7.1	Signaling to support model inference	65

7.2	Consistency aspects in general	66

7.2.1	Consistency of validity area	67

7.2.2	Consistency of RS configuration between model inference and model training	69

7.2.3	Consistency of network-side additional conditions between model inference and model training	70

7.2.4	Consistency of measurements reported between model inference and model training	72

7.3	Consistency of network side conditions for Case 1	72

8	Conclusion	74

9	References	85

10	Appendix A. High accuracy uncertainty and confidence as defined in TS 23.032	85

11	Appendix B. Model performance monitoring approaches	87

11.1.1	Monitoring approach 1 (Label-based): Opportunistic LoS links	88

11.1.2	Monitoring approach 2 (Label-based) for assisted AI/ML positioning	92

11.1.3	Monitoring approach 3 (Label-free): ML model self-monitoring	94

Introduction

In RAN#106 meeting, the WID was revised for Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface [5].

The objective of the normative work is copied below. In this contribution, we focus on the normative work for the use case of positioning accuracy enhancements.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

Integrating AI/ML based positioning methods with existing protocol

In order to specify enhancements to enable AI/ML based positioning, a framework issue to consider is how to integrate the AI/ML based methods with existing protocol. Different phases of AIML operations will map to different procedures and it is important to understand what is already available and what must be introduced to the specifications. Even if RAN2 is the driving force for such new high level procedure, RAN1 needs to understand these procedures to go forward.  In our RAN2 contribution, we present our view for AI/ML positioning integration into the positioning framework.  At a high level, the LCM procedures for each case are model inference, model monitoring and data collection.

Cases with AI/ML model hosted in the UE (Case 1)

Case 1 is a UE based positioning case. The 3GPP positioning framework currently supports two methods for UE based positioning, namely DL-AoD and DL-TDOA based positioning. Additionally, these methods can be initiated either by the network (LMF) via MT LR procedures, or by the UE via MO LR procedures. To support Case 1, an existing procedure can be reused or a completely new procedure with associated AD transfer, capability transfer and location request procedures can be designed.

Assistance data for case 1 was agreed to cover the assistance data for DL TDOA in RAN1#119.  In our view, the agreement is sufficient, and there is no need to consider further methods.

RAN1 assumes Case 1 is supported using UE based DL-TDOA procedures as a foundation. It is up to RAN2 to decide whether a set of new procedure is to be defined for Case 1, or extension to DL-TDOA suffices. RAN1 confirms the understanding with RAN2.

Case with AI/ML model hosted in the TRP/gNB (Case 3a)

In RAN1#118, uplink positioning for case 3a was discussed with the following outcome:

Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

RAN3 further confirmed that legacy methods were re-used for case 3a:

RAN3 agreement, RAN3#126 Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa.

Even though some measurement reporting has been agreed, it was not clarified what legacy method should be used as a baseline for Case 3a.  For this RAN1 should consider the following:

- The Rel-18 study item phase of the AI/ML positioning work focused on time-based methods.

- Positioning methods and enhancements introduced in Rel-18 study item have not been investigated thoroughly for specification.

Based on the lack of studies on angle-based methods, it is reasonable to focus only on time-based positioning methods that use the reported AI/ML model output as model input, i.e. UL-DTOA and Multi-RTT. Thus the above agreement and conclusion are sufficient for defining model output types of assisted positioning.  Other types of measurements, such as angle measurements, were not thoroughly evaluated in Rel-18 SI and should not be considered for Rel-19.

Based on the agreements, RAN1 should assume that the procedures for UL-RTOA and multi RTT should be the basis for Case 3a.  Some of the signalling requests and reports in the procedure may need further extension to properly support the training and operation of an AI model in the gNB/TRP.

RAN1 assumes Case 3a is supported using UL-TDOA or multi RTT procedure as a foundation. It is up to RAN3 to decide whether a set of new procedure is to be defined for Case 3a, or extension to existing procedure suffices. RAN1 confirms the understanding with RAN2 and RAN3.

Case with AI/ML model hosted in the LMF (case 3b)

For model inference of Case 2b and 3b, the LMF requests measurements and runs its own model.

For Case 3b, the framework in RAN3 is flexible enough to be reused. RAN1 agreed to support both sample and path based timing measurements in RAN1#119:

Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

However, for sample-based reporting, the gNB  behaviour should be clarified, i.e. how the LMF requests sample-based measurements.  From the RAN1 perspective, it should be sufficient to assume it is possible for the LMF to specifically request sample-based measurements and is able to differentiate legacy measurement reports (i.e., path-based) from Case 3b reports with sample-based measurements. We discuss the use of such indicators in section 3.1. Note that how the procedure is implemented is up to RAN3.

RAN1 assumes that for Case 3b with sample-based reporting enhancements, measurement report with sample-based reporting can be differentiated by the involved nodes (LMF, gNB) from the legacy UL RTOA measurement requests / reports. RAN1 confirm the understanding with RAN2 and RAN3.

Signaling enhancement for model inputs

In this Section, we investigate the issues related to model input. We first note that, for the following two positioning cases with UE-side model:

- (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

- (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

the measurement generation process is collocated with the AI/ML models. The exact details of the model inputs are largely implementation issues. There is no need to specify model input for model inference stage. Whether model input need any specification work for training data collection can be further discussed.

For Case 1/2a (i.e., UE-side model), channel measurement generation process is collocated in the same node with the AI/ML models. The exact details of the model inputs are implementation issues at least for model inference.

For the two direct AI/ML positioning cases:

- (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

the measurements obtained at one node (i.e., gNB or UE) need to be signaled to a second node (i.e., LMF) that applies the AI/ML models. Thus model input need to be specified at least for Case 3b/2b, and there is a need to clarify the model input formats.

For Case 3b (1st priority) and Case 2b (2nd priority), due to the LMF sided model, there is a need to clarify the target model input formats to be supported for the AI/ML positioning models.

For the 1st priority Case 3a, the need to specify channel measurements for model input depends on the node that hosts the AI/ML model, as discussed in section 3.7. If the need is confirmed by RAN3, the same design choices made for Case 2b/3b can be applied to Case 3a as well. Thus, RAN1 can proceed with the model input discussion on Case 2b/3b.

Remaining issues for sample-based measurements for model input

In RAN1#116, it was agreed to investigate the way timing information was conveyed, i.e., sample-based vs path-based measurements:

Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

During RAN1#119, the use of sample based reporting was agreed, together with path based reporting, as the supported options for case 3b:

Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

The remaining issues to be discussed are thus:

- Parameter values for Nt, Nt', k.

- Support of sample based reporting for UE measurement reports.

- How the LMF can request sample or path based measurement reports.

Value range for parameters {Nt, Nt', k}

Another open issue in the agreed definition of sample-based measurement is the value range of parameters Nt, Nt' and k for sample-based measurement.

For Case 3b, parameters Nt, Nt' and k are selected by LMF and sent to gNB in the measurement request, and the gNB provide measurement report based on the {Nt, Nt', k} values by LMF. The value range of parameters {Nt, Nt', k} should not be excessive, because the LMF need to collect data and train a different model for different parameter settings, for example, a different model for each combination of {Nt, Nt', k}.

From the available range for Nt and Nt’ agreed in RAN1#119, The evaluation results in Rel-18 study item phase as well as Rel-19 show that the maximum value of Nt can be set as 64, since it shows no tangible performance degradation compared to 128.

From the candidate values agreed in RAN1#119 for Nt (Nt = {32, 64, 128} ), Nt = 128 does not provide a significant gain.

From the candidate values agreed in RAN1#119 for Nt (Nt = {32, 64, 128} ), support Nt = {32, 64}.

For the values of Nt’, it was agreed that Nt'<=24. The minimum value of Nt can be set as 9, the same as the maximum number of paths in the existing measurement report. As seen in the study from Rel-18, there are benefit in reporting up to the full 24 samples (the SI reports shows benefits up to Nt’=64)

For the different combination of Nt', Nt', at least the legacy for  Nt'= 9 values should be supported. Since larger Nt’ (e.g., Nt’=64) values have been shown to provide performance benefit, the largest agreed value Nt’=24 should be supported. In summary, the following parameter combinations can be supported:

  - For Nt = 32: {Nt, Nt'} = {32, 24}, {32, 9};

  - For Nt = 64: {Nt, Nt'} = {64, 24}, {64, 16}, {64, 9};

From the candidate values agreed in RAN1#119 for Nt' (Nt' <=24), Nt' values {9, 16, 24} are supported.

For {Nt, Nt'}, support these combinations: {32, 24}, {32, 9}; {64, 24}, {64, 16}, {64, 9}.

The exact {Nt, Nt'} value to configure may depends on the deployment environment (e.g., heavily NLoS or not), system parameters including SRS configuration (e.g., carrier frequency, SRS bandwidth), and the k values for timing granularity. Exactly how to select the {Nt, Nt'} value from the possible combinations can be up to LMF implementation.

In terms of k values, for legacy measurement report for DL-TDOA and multi-RTT,

- Up to Rel-17, the set of possible k values are: {0…5}. That is, the timing measurement granularity ranges from Tc (i.e., 0.51 ns) to 32×Tc (i.e., 18.75 ns).

- In Rel-18, new ReportingGranularityfactor values are added for Carrier Phase Positioning: {-6 … -1}. That is, the new timing measurement granularity ranges from Tc/64 (i.e., 0.008 ns) to Tc/2 (i.e.,0.25 ns).

Note: Tc is the basic NR timing unit, and Tc = 1/ (Δ𝑓max ⋅ Nf) = 1/(480*103*4096) = 0.5086 (ns).

According to the existing specification (see extracted texts for DL-TDOA below), the Rel-16 ReportingGranularityfactor and Rel-18 timingReportingGranularityFactorExt are not used at the same time, i.e., only one is included.

TS 37.355 v18.2.0 NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE { maxDL-PRS-RSTD-MeasurementsPerTRP-Pair-r16 INTEGER (1..4) OPTIONAL, -- Need ON timingReportingGranularityFactor-r16 INTEGER (0..5) OPTIONAL, -- Need ON ..., [[ measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 ENUMERATED { n0, n2, n3, n4, n6, n8, ... } OPTIONAL, -- Need ON reducedDL-PRS-ProcessingSamples-r17 ENUMERATED { requested, ... } OPTIONAL, -- Need ON lowerRxBeamSweepingFactor-FR2-r17 ENUMERATED { requested } OPTIONAL -- Need ON ]], [[ timingReportingGranularityFactorExt-r18 INTEGER (-6..-1) OPTIONAL, -- Need ON nr-DL-PRS-MeasurementTimeWindowsConfig-r18 NR-DL-PRS-MeasurementTimeWindowsConfig-r18 OPTIONAL -- Need ON ]] } timingReportingGranularityFactor, timingReportingGranularityFactorExt This field specifies the recommended reporting granularity for the DL RSTD measurements. Value (0..5) corresponds to (k0..k5) and value (-6..-1) corresponds to (kMinus6..kMinus1) used for nr-RSTD and nr-RSTD-ResultDiff in NR-DL-TDOA-MeasElement. The UE may select a different granularity value for nr-RSTD and nr-RSTD-ResultDiff. The timingReportingGranularityFactorExt should not be included by the location server and shall be ignored by the target device if timingReportingGranularityFactor is included.The timingReportingGranularityFactor should not be included by the location server and shall be ignored by the target device if timingReportingGranularityFactorExt is included..

For AI/ML based positioning, vast majority of companies used k=4 (i.e., sampling period = 16×Tc) in Rel-18 study item [4]. The Rel-18 evaluations demonstrated that the AI/ML based positioning was able to achieve sub-meter positioning accuracy with k=4.

Clearly, the ultra-fine timing granularity introduced for Rel-18 CPP are excessive and unnecessary for AI/ML positioning. There is no justification to duplicate the signaling for CPP over to AI/ML based positioning even though k={-6,..,-1} can continue to be used for measurement reporting of legacy methods DL-TDOA and multi-RTT.

Even for the 6 values {0..5} up to Rel-17, there is no justification to support all 6 values. Having a wide range of k values complicates the AI/ML LCM. For each k values, the full set of AI/ML procedures have to be supported separately, including training data collection, model training, model monitoring, model update, etc.

In our view, it is preferrable to support a small set of k values in Rel-19, which is the first release to adopt AI/ML based positioning. In fact, it may be sufficient to support one k values in Rel-19, k=4, which has been thoroughly evaluated in Rel-18 study item by numerous participating companies. If more options are needed considering FR1/FR2 and different PRS bandwidth, it can be expanded to support four k values, for example, k={2,3,4,5}. Future releases can consider more options of k values if proven necessary for the desired positioning accuracy.

For Case 3b (and 3a if needed), a small number of k values, e.g., k={4}, k={3,4}, or k={2,3,4,5}, is supported for timing reporting granularity factor in Rel-19.

During RAN1#119, it was agreed that the window parameter configuration values Nt, Nt', k are signalled to gNB via NRPPa, but may be changed by the gNB. In that case, the actual values used by the gNB/TRP for  Nt, should be sent to the LMF in the measurement report (note that parameter k value is already reported back). The LMF can then better judge the compatibility of the measurement report with its model.

For Case 3b (and 3a if needed), when enhanced measurement are reported (i.e., sample-based reports) from the gNB to the LMF:

  - When the Nt, Nt', k values used to produce the report are different from the ones provided by the LMF in the measurement request, gNB provides in the measurement report to the LMF the Nt value used for the measurement.

  - Note: k value is already supported in the measurement report, and Nt’ can be inferred by the report size.

LMF handling of sample-based or path-based reporting

For case 3b, the LMF handles the AI model, and therefore should be able to request a suitable model input measurement.

For Rel-19 AI/ML based positioning Case 3b, LMF includes an indicator in the measurement request to gNB, where the indicator is on the LMF preferred measurement report: either (a) sample-based measurement, or (b) legacy path-based measurement.

Regarding legacy path-based measurement, there is a question on whether to extend the number of paths to report. In our view, if path-based measurement is used, then the existing maximum of 9 paths (first path + 8 additional paths) is sufficient. If more measurement values are desired, then sample-based measurement can be used instead.

For Rel-19 AI/ML based positioning, legacy path-based measurement (if needed) is used without enhancement.

Finally, we also discussed possibility of fallback between sample-based reporting and legacy RTOA reporting.  In our view, it should be left to the LMF to decide whether to request a fallback, since the LMF model implementation may or may not be able to reuse legacy measurements.  Moreover, the gNB reporting capability for path or sample measurement report may dynamically change due to processing load variations. Hence automatic fallback from one type of measurement to the other cannot be guaranteed.  Hence, it should be enough to rely on the legacy “cause value” mechanism. If the LMF request for sample-based reporting cannot be satisfied, a gNB  can reject the request with a suitable cause value, and the LMF may proceed by subsequently send a new request for legacy RTOA measurements.

For Case 3b measurement reporting, gNB measurement report capability may vary over time.

For Case 3b, automatic fallback from one measurement method to the other cannot be guaranteed.

For Case 3b, do not specify signalling enhancements for fallback between sample-based and path-based measurement.

  - Note: fallback can be supported by existing procedure using cause values.

Total-power PDP report for Rel-19

For a UE or a gNB equipped with multiple receive antenna ports, sources had investigated tradeoffs of using the following two different PDP input options during the SI phase:

- total-power PDP (summed over all antenna ports)

pd≜ahad2

- Multi-port PDP (measurement at each receive antenna port individually)

pa[d]≜had2

It is noted that the reference signals defined for positioning (DL PRS, UL positioning SRS) is single-port at transmit antenna. Hence 'multi-port' in this discussion refers to receive antenna ports only.

One set of evaluation results are copied in Table 1 where the 90%tile positioning errors of using 2-port PDP or the summed PDP are compared. It can be observed that

- The 2-port PDP input type doubles the signal sizes, requires higher computational complexity, and achieves marginal performance improvements when compared to the total PDP input type.

- Using only 1-port to compute PDP discards signal power and radio channel information that is readily available at the measurement node, which causes lower positioning accuracy when compared to the total PDP input type. The positioning accuracy degradation ranges from 11% with very large training datasets to 20% with smaller training datasets.

When compared to the total-power PDP input type, the 2-port PDP input type (1) doubles the signal sizes; (2) requires higher computational complexity; and (3) achieves marginal performance improvements.

When compared to the total-power PDP input type, the 1-port PDP input type (1) discards signal power and radio channel information that is readily available, and (2) achieves lower positioning accuracy.

Table 1 90%tile 2D positioning accuracy (meters) using summed total-power PDP or 2-port PDP inputs for the small models and different training dataset sizes in the {60%, 6m, 2m} InF-DH scenario. Nt=Nt'=256. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6].

| Input | Model complexity [# parameters] | Computational complexity [FLOPs] | 90%tile 2D positioning error [m] with different train set sizes in {60%, 6m, 2m} InF-DH |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 80,000 | 40,000 | 20,000 | 10,000 |
| total-power PDP | 0.36 M | 9 M | 0.43 | 0.51 | 0.66 | 0.86 |
| 2-port PDP | 0.37 M | 11 M | 0.40 | 0.50 | 0.64 | 0.83 |
| 1-port PDP | 0.36 M | 9 M | 0.47 | 0.58 | 0.76 | 1.04 |

It can be concluded that the performance gains of using multi-port PDP do not justify the multiple increases in signaling sizes; and 1-port PDP discards useful information unnecessarily. Therefore, Rel-19 PDP reports should be based on PDP samples containing sample powers from all antenna ports.

Furthermore, the nonzero value positions of the DP report type should be determined based on PDP samples containing sample powers summed from all RX antenna ports.

Additionally, if 1-port is used, there exists the inconsistency issue between training and inference. That is, since it is up to implementation to select the receiver branch(es) in the existing definition of PRS-RSRPP and SRS-RSRPP, the UEs in training data collection and the UEs in inference are likely to use different receiver branch(es). This inconsistency issue does not exist if total-power is used instead.

For the existing SRS-RSRPP definition, the receiver branch(es) to select is based on the first path, see the red-highlighted phrase below. If this is extended straightforwardly to sample-based measurement, then the receiver branch(es) to select is based on the first path for path-based measurement, and based on the first sample for sample-based measurement. However, for AI/ML fingerprinting, which is expected to be deployed in heavy NLOS environment, the first path or the first sample is expected to be weak and noisy, thus unreliable for selecting the strongest receiver branch. If using total-power, there is no need to select receiver branch(es), thus avoiding this issue. Furthermore, using total-power avoids any need to specify that the measurements for additional paths/samples are provided for the same receiver branch(es) as the first path/sample measurement (see blue-highlighted text below).

Accordingly, definition of DP and PDP measurements in 38.215 should be made according to the above for AI/ML positioning.

Regarding PDP for model input, update the DL PRS-RSRPP and UL SRS-RSPP definitions where the path or sample powers are summed over all receive antenna ports, i.e., using total-power PDP.

Existing definitions in 38.215:

5.1.35	DL PRS reference signal received path power (DL PRS-RSRPP)

| Definition | DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE for DL PRS-RSRPP measurements, the reported DL PRS-RSRPP value included in the higher layer parameter NR-DL-AoD-MeasElement for the first and additional measurements shall be provided for the same receiver branch(es) as applied for DL PRS-RSRP measurements. |
| --- | --- |

5.2.6	UL SRS reference signal received path power (UL SRS-RSRPP)

| Definition | UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry the received UL SRS signal configured for the measurement, where UL SRS-RSRPP for 1st path delay is the power contribution corresponding to the first detected path in time The reference point for UL SRS-RSRPP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: based on the combined signal from antenna elements corresponding to a given receiver branch - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements: - The reported UL SRS-RSRPP value for the first and additional paths shall be provided for the same receiver branch(es) as applied for UL SRS-RSRP measurements, or - The reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches and the reported UL SRS-RSRPP for the additional paths shall be provided for the same receiver branch(es) as applied UL SRS-RSRPP for the first path. |
| --- | --- |

Analysis of supporting phase information measurement

In RAN1#116, the following agreement was made to investigate whether phase information should be supported as a part of model input, for example, as list of phase information as in CIR or one phase value for the first detected path only.

Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

In the sub-sections below, analysis is provided for the following aspects:

- Inapplicability of Rel-18 carrier phase positioning;

- Issues with non-spatial-dependent initial phases;

- Difficulty to align phase measurements between model training and model inference;

- Signaling overhead

Considering all the pros and cons of phase measurement as model input, we conclude that it is not feasible nor necessary to support phase information for determining model input, including CIR and single phase value for first path.

Do not support phase information for determining model input, including CIR and single phase value for first path/sample.

Inapplicability of Rel-18 carrier phase positioning

During RAN1#116bis, there was a discussion on supporting Rel18 CPP measurements (DL RSCPD, DL RSCP, UL RSCP) for AI/ML positioning as model input. Since release 18 positioning was specified in parallel to the AI/ML study item phase, CPP was not evaluated during the Rel-18 AI/ML study. Moreover, CPP was specified for environement with very high line of sight probability. During Rel18 CPP specification, it was concluded  that LOS was critical to proper operation of CPP.  Additionally, In AI/ML cases, the TRP/UE links are generally NLOS, and the phase information of the first path/sample does not carry much information of the UE position.  Additionally, NLOS phase information is very sensitive to even minimal environement changes, such as changes in the nature of the material reflecting the path.  Finally, CPP requires PRU to be operating next to the UE during both training and inference, which adds to deployment cost and signalling overhead.

Rel-18 CPP measurements (DL RSCPD, DL RSCP, UL RSCP) assume LoS environment, which is contradictory with the target use case of Rel-19 AI/ML based positioning.

Single phase value for first path/sample (e.g., DL RSCPD, DL RSCP, UL RSCP) brings no observable performance benefits to PDP, but incurs substantially high deployment cost and signalling overhead.

Issues with random initial phase

The clock and/or frequency generation circuits are essential components in modern digital wireless communications systems. The circuits are used to drive reference waveform to demodulate signals received over the air. Due to the random states of clock/frequency generation circuits in the measurement radio node, the phase of the ML measurement data such as the time domain channel impulse responses may contain random and unhelpful information to AI/ML models. For the AI/ML positioning application, this means for UE revisiting the exact same location, the TD CIR sample sequences may still differ because the state of the clock/frequency generation circuits may be at different state. A TD CIR sample ha(1)[d] measured at a first visit and ha(2)d measured at a second visit may be different because of the different states of clock/frequency generation circuits.

More specifically, let the sequence of Nt' complex-valued TD CIR samples measured on receive antenna port a be

had0, had1, …, ha[dNt'−1]

where d0, d1, …, dNt'−1 are the sampling time locations of the TD CIR samples. The sampling time locations may be on a regular sampling grid with integer values or off the regular grid with non-integer values as discussed in the last section. The complex CIR samples can be equivalently represented by the power and phase parts:

pad0,pad1, …, pa[dNt−1]

θad0, θad1, …, θa[dNt−1]

where had=pad⋅expjθad, for d=d0, d1, …, dNt'−1.

The above discussion indicates that the initial phase value θad0 may in fact be random and contains no useful spatial dependent information. An AI/ML model trained with samples containing such spurious values will nonetheless learn to associate the target positions with these random initial phases in the training dataset. When fed with new measurements with new random phases during inference, the model will be confused and generates inaccurate position estimates.

For instance, we trained a Nt=256, Nt'=128 CIR model using a training dataset of 40,000 samples with the measured phases (i.e., containing the random initial phases). In Table 2, we tested the trained model under two different conditions:

- When the trained model is tested with consistent initial phase (that is, the initial phase values for different UE positions reflects only the spatial differences), the model achieves a 90%tile 2D UE positioning error of 0.38 m.

- When the trained model is tested with random initial phase (that is, the initial phase values for different UE positions reflects the spatial differences and random phases), the 90%tile 2D UE positioning error increases by 20% to 0.46 m.

Table 2 Comparison of 2D UE positioning errors Δ90% (in meters) for a Nt=256, Nt'=128 model trained without random initial phase and (1) tested without random initial phases or (2) tested with random initial phases in the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6].

| CDF Percentile | UE 2D positioning errors [m] in {60%, 6m, 2m} Model I trained with 40,000 CIR samples without random θad0 |  |
| --- | --- | --- |
|  | Tested without random θad0 | Tested with random θad0 |
| 50 | 0.19 | 0.23 |
| 67 | 0.25 | 0.30 |
| 80 | 0.31 | 0.38 |
| 90 | 0.38 | 0.46 |

The initial phase values of measured CIR samples may be random and contains no useful spatial dependent information for an AI/ML model to learn the association between measured CIR samples and the target positions. Without addressing such spurious information during training and inference, fingerprinting ML models using the CIR inputs can produce inaccurate position estimates.

Before CIR can be adopted as model input, RAN1 need to investigate whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits.

While the initial phase θad0 may be random and contains no useful channel dependent information, the relative phases of the samples

∆θad=θad−θad0,  for d=d1, d2, …, dNt−1

still contain useful information relating the channel and the sampling time. Hence, one way to circumvent the impact of the random initial phases is to scrub it from the model input. That is, instead of recording/signaling Nt' values of powers and Nt' values of phases, only 2Nt'−1 values are signaled/recorded:

pad0,pad1, …, pa[dNt−1]

θad1, ∆θad2, …, ∆θa[dNt−1]

The phase of the initial sample θad0 is taken as zero and not signaled. Equivalently, this is just applying a common phase de-rotation to the complex-valued CIR samples to set the initial phase to zero:

pad0, e−θad0had1, …,e−θad0ha[dNt'−1]

In Table 3, we compared the 90%tile 2D UE positioning errors Δ90% (in meters) for two sets of models:

- The first set of models are trained and tested with CIR samples containing no random initial phases. Note that it’s not possible to collect such type of CIR samples in reality because of the random states of the clock and/or frequency generation circuits discussed in the above.

- The second set of models are trained and tested CIR samples containing relative phases as just described.

It can be observed that the models trained and tested CIR relative phase samples achieve essentially identical positioning accuracy (up to simulation noises) as the models trained and tested with idealized (but not physically feasible) CIR samples.

CIR samples containing relative phases can be used as inputs to AI/ML models to circumvent the impact of random initial phase values of measured CIR samples.

Table 3 Comparison of 90%tile 2D UE positioning errors Δ90% (in meters) for (1) models trained and tested with CIR samples containing no random initial phases and (2) models trained and tested with CIR samples containing relative phases in the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6].

| Nt | Nt' | 90%tile 2D positioning error [m] in {60%, 6m, 2m} InF-DH Model I trained with 40,000 CIR samples |  |
| --- | --- | --- | --- |
|  |  | Trained and tested without random initial phases | Trained and tested with relative phases |
| 256 | 128 | 0.38 | 0.39 |
| 256 | 64 | 0.43 | 0.44 |
| 128 | 64 | 0.48 | 0.46 |
| 256 | 32 | 0.54 | 0.55 |
| 128 | 32 | 0.53 | 0.55 |
| 64 | 32 | 0.67 | 0.66 |
| 256 | 16 | 0.68 | 0.69 |
| 128 | 16 | 0.64 | 0.65 |
| 64 | 16 | 0.70 | 0.71 |
| 256 | 9 | 0.82 | 0.84 |
| 128 | 9 | 0.82 | 0.81 |
| 64 | 9 | 0.80 | 0.80 |

If the relative phase described above is to be supported, then new measurement types needs to be introduced in 38.215. An example is shown below and called UL RSRCP, which is for UL measurements based on SRS. Similar new measurement type is needed for DL PRS as well.

UL reference signal relative carrier phase (RSRCP) is defined as a sequence of relative phase values of the channel response derived from the resource elements carrying sounding reference signals (SRS) configured for the measurement. For the j-th path, the relative phase is the difference between the phase of the j-th path and the phase of the first path.

It is noted that the UL RSRCP illustrated above needs to be added in addition to the existing UL reference signal carrier phase (RSCP) introduced in Rel-18, which is only for the phase of the first detected path delay. Related to the new measurement type UL RSRCP, standardization effort is also needed in RAN1/RAN2/RAN3 for signaling support for training data collection and model inference, and potentially in RAN4 for requirements. Similar standardization effort is needed for the corresponding DL PRS measurement as well. RAN1 needs to take these into account in determining whether to support CIR in Rel-19.

Furthermore, the analysis above does not consider inter-path phase difference caused by the TRP’s and the UEs antenna patterns. As discussed in section 3.2.3, it is unclear how to remove such inter-path phase difference between model training and model inference, especially the phase offset caused by UE Tx or Rx antenna.

To decide whether to support phase information to enable CIR as model input, RAN1 should weigh the small positioning accuracy improvement against the standardization effort, the signaling overhead, and the difficulty to align phase information between training and inference.

If phase information is supported to enable CIR as model input, the phase values are reported in the format of relative phase.

Difficulty to align phase measurements between model training and model inference

Assuming a magnitude-phase measurement is used as model input, e.g., a complex CIR or even a single path, some ambiguity will need to be resolved. In the downlink, the phase component of a received path consists of the following:

θue=θ(d1u)+Δ1(αTRP)+Δu(βue)

In this equation, we can see that the receive phase is made of:

- The channel path phase θ(d1u), which is a function of the distance between the TRP and the UE.

- The TRP phase at the time of transmission Δ1αTRP, which includes a random phase rotation caused by the free-running oscillator, and a deterministic rotation caused by the beamforming weights and the Tx antenna pattern. Thus the initial phase depends on the direction of departure of the path.

- The UE phase at the time of reception Δu(βue), which includes a random phase rotation caused by the free-running oscillator, and a deterministic rotation caused by the beamforming weights and the Rx antenna pattern. Thus the initial phase depends on the direction of arrival of the path.

Let’s assume that the model input is the RSCP/RSCPD measurement defined in release 18, i.e., the (complex) phase/ phase difference for the first path. Carrier phase positioning specified in release 18 measures the phase difference between two DL PRSs received from two TRPs:

θuediff=θd1u+Δ1αTRP1+Δuβue− θd2u+Δ2αTRP2+Δuβue

Doing so cancels out the UE’s own random phase error contribution to the phase error. We can assume there is no UE phase error contribution left if the UE knows its antenna pattern’s phase offset and can remove it. The remaining phase difference becomes:

θuediff=θd1u+Δ1αTRP1− θd2u+Δ2αTRP2

To cancel out the TRPs contribution to the phase error, a PRU is used to collect a measurement from each TRP and form its own phase difference:

θPRUdiff=θd1,PRU+Δ1PRUαTRP1− θd2,PRU+Δ2PRUαTRP2

The assumption is that the PRU allows the estimator to perfectly know the distance between the TRP and the PRU, so that the differential phase errors between the TRPs can be estimated as:

θPRUdiff−θd1,PRU+θd2,PRU=Δ1PRUαTRP1− Δ2PRUαTRP2

The final phase difference then can be expressed as following and can be used to find θd1u−θd2u:

θuediff=θd1u−θd2u+θPRUdiff−θd1,PRU+θd2,PRU

In the above, it is assumed that Δ1PRUαTRP1− Δ2PRUαTRP2=Δ1αTRP1−Δ2αTRP2. If the PRU and the UE are collocated well enough, and the measurements are taken very close in time, the PRU-estimate of the differential phase error between TRPs corresponds to the TRP phase error experienced by the UE. for this reason, release 18 specified a measurement time window for RSCP/RSCPD measurements. Hence, if the Rel-18 measurement is used for measurement input, PRUs need to assist the measuring UE during inference, since the phase errors from TRPs are time-dependent.

For the RSCPD measurement PRU assisted phase error compensation requires PRUs to be used during training and inference, further increasing the deployment cost and signaling overhead.

During RAN1#116, it was also discussed to use inter-path phase information to resolve the issue of random phase at transmitter and receiver, see also analysis in section 3.2.2. Inter-path phase information will indeed remove the random phase error caused by the transmitter and receiver oscillators. What remains is the inter-path phase difference caused by the TRP’s and the UEs antenna patterns.  Phase errors due to the TRP antenna will be the same during training and inference and thus can be resolved, but the UE antenna pattern will be difficult to keep consistent between inference and training, since the UE’s antenna pattern as well as its relative orientation toward the TRP may be different between training and inference. One other alternative would be to remove the phase difference between path caused by the antenna, which means the UE phase error caused by the Rx (Tx) antenna must be known to the model, for each direction of arrival (departure).

For inter-path phase measurements, the model input at training and inference must know the UE TX or RX antenna phase contribution, either by reporting the antenna phase contribution to the model, removing it, or assuming the same antenna pattern is used during inference and training.

It is unclear how a mobile UE would know the direction of departure / arrival and associated phase offset for each path toward a given TRP.

Signaling size vs positioning accuracy tradeoff analysis

The complex-valued multi-port CIR samples have been found in the Rel-18 SI to achieve high positioning accuracy. However, these CIR samples require significantly larger signaling sizes than the total-power PDP. To reduce signaling sizes, many different down-sampling techniques such as truncation and sub-sampling have been investigated during the SI phase. It is therefore necessary to investigate the trade-off between signaling size requirements and achievable positioning accuracy of various combinations of sample lengths, sub-sample sizes and with CIR or PDP.

In the following, we present two signaling size vs positioning accuracy trade-off comparison.

- In Section 3.2.4.1, we use the number of real numbers to be signaled as a proxy of the signaling size.

- In Section 3.2.4.2, we construct a more detailed signaling size estimation based on specific feedback formats.

From both signaling sizes vs positioning accuracy trade-off analysis exercises, we arrive at the following observation.

For small or moderate signaling sizes, PDP and DP samples can achieve better positioning accuracy than CIR samples at the same or smaller signaling size. Multi-port CIR samples can achieve higher positioning accuracy only with very large signaling requirements.

For small or moderate signaling sizes, PDP and DP samples can achieve better positioning accuracy than CIR samples at the same or smaller signaling size as well as with substantially lower AI/ML model complexity.

For most given 90%tile 2D UE positioning error requirements, the DP samples requires the smallest signaling sizes.

Furthermore, the assumption of two receive antenna ports for the UE was a simplifying assumption made for Rel-17 Positioning WI. For the Rel-19 AI for PHY, it may be necessary to consider four receive antenna ports for the typical NR UEs. This would double the required numbers of real values to represent the multi-port CIR again for Case 2b (2nd priority).

For Case 3b (1st priority), a typical gNB/TRP may be equipped with dozens of receive antenna ports. Forwarding such large multi-port CIR samples to LMF will have significantly negative impact on radio and core networks.

Multi-port complex-valued CIR samples for both Case 3b (1st priority) and Case 2b (2nd priority) require very large signaling sizes, which can cause significantly negative impacts on the radio and core networks.

Based on the extensive evaluation and analysis presented in this section, we propose

RAN1 to down-prioritize the signaling approach(es) and/or measurement definitions to support CIR model input types for Case 3b (1st priority) and Case 2b (2nd priority).

Simple signaling size comparison based on the number of real numbers to signal

In this section, we make a first quick comparison of feedback sizes and achievable positioning accuracy based on the number of real numbers needed in the signaling. An NR radio node is normally equipped with multiple RX antenna ports. AI/ML models with multi-port CIR inputs have been evaluated by several companies during the SI phase. As shown below, multi-port CIR can enable AI/ML models to achieve high positioning accuracy but will also require large signaling overhead. In the following, we investigate whether this accuracy vs overhead tradeoff can be improved by reducing the dimensions of the multi-port CIR inputs.

We consider two dimension-reduction approaches to reduce multi-port CIR inputs to a single vector:

- Strongest port CIR

The measuring radio node computes the RSRP of each RX antenna ports and signals CIR samples only for the port with the strongest received power.

- First principal component (1st PC) CIR

Let the Nt'×A matrix H collects the TD CIR samples from all antenna ports. The first principal component is given by Hv, where the length-A vector v is the eigenvector corresponding to the largest eigenvalue of HHH.

Our positioning accuracy evaluation results for 2-port CIR, strongest port CIR, 1st PC CIR and PDP inputs are shown in Table 4. In this comparison, we classify different combinations of sample lengths, sub-sample sizes and with CIR or PDP based on the number of real numbers to represent the samples. Similarly, in Table 5, we compare the positioning accuracy vs signaling sizes with dynamic TRP selection for Ntrp'= 18, 9, 6 or 3. The total number of real numbers to signal is given by Ntrp'×Nt' for PDP and 4×Ntrp'×Nt' for 2-port CIR.

- For instance, a length Nt=128 2-port CIR with Nt'=32 sub-samples require 128 real numbers per reported TRP to represent the complex-valued CIR samples.

- For the same number of real numbers in signaling, the strongest port CIR or the 1st PC CIR samples can be of length Nt=256 with Nt'=64 sub-samples per reported TRP.

- For the same number of real numbers in signaling, the PDP samples can be of length Nt=256 with Nt'=128 sub-samples per reported TRP.

It is then a fair comparison to check which approach achieves better performance when they require the same number of real values to represent the samples. From the table, we make the following observations:

- For small or moderate signaling sizes, PDP samples can achieve better positioning accuracy than 2-port CIR samples at the same or smaller signaling sizes. Using PDP samples, a 90%tile 2D positioning error of 0.52 m can be achieved.

- The higher positioning accuracy achieved by 2-port CIR is only possible with very large signaling sizes.

- While dimension reduction techniques reduce the signal sizes of CIR samples when compared to using the multi-port CIR directly, the achievable positioning accuracy is also compromised. Overall, applying dimension reduction to reduce the signaling sizes of the multi-port CIR samples does not appear to improve its accuracy vs overhead tradeoff.

  - The first principal component approach enables marginally better AI/ML model positioning accuracy than the strongest port selection approach.

From a model input design point of view, one can conclude from these evaluation results that

- With small to moderate number of samples available to an ML model, the total-power PDP input type is a very helpful induction bias to impose on the ML model based on human domain knowledge that the sample powers contain more important information about the UE positions.

  - Using the real-valued PDP inputs also reduces the AI/ML model complexity substantially as shown in Table 6.

- Only with very large number of samples available to the ML model, the model can start to tease out how to use the additional information in the sample phases on its own.

With small to moderate number of samples available to an ML model, the total-power PDP input type is a very helpful induction bias to impose on the ML model based on human domain knowledge that the sample powers contain more important information about the UE positions. It’s only with very large number of samples that the model can start to tease out how to use the additional information in the sample phases on its own.

Dimension reduction techniques can be used to reduce the signal sizes of multi-port CIR samples. But the achievable positioning accuracy is also compromised. The overall accuracy vs overhead tradeoff situation of CIR samples is not improved by the two considered dimension reduction techniques.

Table 4 Comparison of 90%tile 2D UE positioning errors Δ90% (in meters) for 2-port CIR, 1-port CIR and PDP using different combination of sample lengths and sub-sampling sizes in the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6] and Ntrp'=Ntrp= 18.

| Number of real values | 2-port CIR |  |  | 1st PC CIR |  |  | Strongest port CIR |  |  | PDP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Nt | Nt' | Δ90% | Nt | Nt' | Δ90% | Nt | Nt' | Δ90% | Nt | Nt' | Δ90% |
| 1024×18 | 256 | 256 | 0.37 |  |  |  |  |  |  |  |  |  |
| 512×18 | 256 | 128 | 0.38 | 256 | 256 | 0.49 | 256 | 256 | 0.48 |  |  |  |
|  | 128 | 128 | 0.44 |  |  |  |  |  |  |  |  |  |
| 256×18 | 256 | 64 | 0.43 | 256 | 128 | 0.50 | 256 | 128 | 0.50 | 256 | 256 | 0.51 |
|  | 128 | 64 | 0.48 | 128 | 128 | 0.62 | 128 | 128 | 0.60 |  |  |  |
|  | 64 | 64 | 0.67 |  |  |  |  |  |  |  |  |  |
| 128×18 | 256 | 32 | 0.54 | 256 | 64 | 0.54 | 256 | 64 | 0.57 | 256 | 128 | 0.52 |
|  | 128 | 32 | 0.53 | 128 | 64 | 0.62 | 128 | 64 | 0.63 | 128 | 128 | 0.56 |
|  | 64 | 32 | 0.67 | 64 | 64 | 0.83 | 64 | 64 | 0.88 |  |  |  |
|  | 32 | 32 | 1.60 |  |  |  |  |  |  |  |  |  |
| 64×18 | 256 | 16 | 0.68 | 256 | 32 | 0.66 | 256 | 32 | 0.68 | 256 | 64 | 0.52 |
|  | 128 | 16 | 0.64 | 128 | 32 | 0.69 | 128 | 32 | 0.70 | 128 | 64 | 0.57 |
|  | 64 | 16 | 0.70 | 64 | 32 | 0.84 | 64 | 32 | 0.84 | 64 | 64 | 0.71 |
|  |  |  |  | 32 | 32 | 1.80 | 32 | 32 | 1.81 |  |  |  |
| (36 or 32) ×18 | 256 | 9 | 0.82 | 256 | 16 | 0.80 | 256 | 16 | 0.85 | 256 | 32 | 0.58 |
|  | 128 | 9 | 0.82 | 128 | 16 | 0.78 | 128 | 16 | 0.83 | 128 | 32 | 0.59 |
|  | 64 | 9 | 0.80 | 64 | 16 | 0.87 | 64 | 16 | 0.88 | 64 | 32 | 0.72 |
|  |  |  |  |  |  |  |  |  |  | 32 | 32 | 1.17 |
| (18 or 16) ×18 |  |  |  | 256 | 9 | 0.97 | 256 | 9 | 1.01 | 256 | 16 | 0.69 |
|  |  |  |  | 128 | 9 | 0.95 | 128 | 9 | 0.99 | 128 | 16 | 0.67 |
|  |  |  |  | 64 | 9 | 0.99 | 64 | 9 | 0.99 | 64 | 16 | 0.75 |
| 9×18 |  |  |  |  |  |  |  |  |  | 256 | 9 | 0.82 |
|  |  |  |  |  |  |  |  |  |  | 128 | 9 | 0.81 |
|  |  |  |  |  |  |  |  |  |  | 64 | 9 | 0.82 |

Table 5 Comparison of 90%tile 2D UE positioning errors Δ90% (in meters) for 2-port CIR and PDP using different combination of sample lengths and sub-sampling sizes in the {60%, 6m, 2m} InF-DH scenario. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6] and dynamic TRP selection with Ntrp'= 18, 9, 6 or 3.

| Number of real values | 2-port CIR |  |  |  | PDP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ntrp' | Nt | Nt' | Δ90% | Ntrp' | Nt | Nt' | Δ90% |
| 18432 | 18 | 256 | 256 | 0.37 |  |  |  |  |
| 9216 | 9 | 256 | 256 | 0.95 |  |  |  |  |
| 6144 | 6 | 256 | 256 | 1.39 |  |  |  |  |
| 4608 |  |  |  |  | 18 | 256 | 256 | 0.51 |
| 3072 | 3 | 256 | 256 | 2.18 |  |  |  |  |
| 2304 |  |  |  |  | 9 | 256 | 256 | 1.07 |
| 1536 |  |  |  |  | 6 | 256 | 256 | 1.55 |
| 768 |  |  |  |  | 3 | 256 | 256 | 2.16 |

Table 6 Complexity comparison for models with (1) 2-port CIR, (2) 1-port CIR, and (3) total Rx power (1-port) PDP inputs.

| 2-port CIR |  | 1-port CIR |  | (1-port/total) PDP |  |
| --- | --- | --- | --- | --- | --- |
| # paras | FLOPs | # paras | FLOPs | # paras | FLOPs |
| 0.73 M | 32 M | 0.52 M | 21 M | 0.36 M | 9 M |

Signaling sizes based on simple direct sample timing bitmaps

The signaling of one TRP link for the DP input type consists of one part:

- Part 1: a bitmap of length Nt to indicate locations of non-zero samples

  - This part is to support sub-sampling the TD measurement samples.

    - The number of indicated non-zero samples in this bitmap is denoted by Nt'.

  - For PDP and CIR, if sub-sampling is not configured, then this part is not needed and Nt'= Nt in the following.

The signaling of one TRP link for the PDP input type consists of Part 1 as defined above and

- Part 2: Nt' powers (or magnitudes) of non-zero samples starting from the lowest tap indices.

The signaling of one TRP link for the CIR input type consists of Part 1 and Part 2 as defined above as well as

- Part 3: Nt' phase of non-zero samples starting from the lowest tap indices.

If the signaling contains feedbacks for NTRP links, the number of bits for each part is

- Part 1: NTRP × Nt bits

- Part 2: NTRP × Nport × Nt' × Breal bits

- Part 3: NTRP × Nport × Nt' × Breal bits

where

- NTRP = 18 or dynamic TRP selection to 9, 6, 3 TRPs

- Nport = 2 for 2-port CIR and Nport = 1 for PDP and 1st PC CIR

- Breal = 8 bits

That is, for one positioning request, the total numbers of feedback bits for reporting measurement are

- DP

  - NTRP × Nt bits

  - Optional: NTRP × (Nt + Breal) bits (including the RS RSRP for the TRP links)

- PDP

  - NTRP × Nt × Breal bits if sub-sampling is not configured

  - NTRP × (Nt + Nt' × Breal) bits if sub-sampling is configured

- 2-port CIR

  - NTRP × 4 × Nt × Breal bits if sub-sampling is not configured

  - NTRP × (Nt + 4 × Nt' × Breal) bits if sub-sampling is configured

  - Note: factor 4 is used above since CIR uses 2-port and complex values

- 1st PC CIR

  - NTRP × 2 × Nt × Breal bits if sub-sampling is not configured

  - NTRP × (Nt + 2 × Nt' × Breal) bits if sub-sampling is configured

  - Note: factor 2 is used above since CIR uses complex values

In Figure 1, we show the scatter plot of feedback sizes (i.e., number of bits to report measurements) and the achieved 90%tile 2D UE positioning errors for different AI/ML model input types with the following different combinations of lengths, sub-sampling sizes and TRP down-selection [6]:

- CIR/PDP

  - Truncation to Nt= 256, 128, 64, 32 taps without sub-sampling

  - Nt= 256-sample with Nt'= 128, 64, 32, 16, 9 sub-samples

  - Nt= 128-sample with Nt'= 64, 32, 16, 9 sub-samples

  - Nt= 64-sample with Nt'= 32, 16, 9 sub-samples

- DP

  - Truncation to Nt=256, 128, 64 taps with Nt' non-zero samples

  - With magnitude 1 or optionally with RS RSRP

- Dynamic TRP selection to 9, 6, 3 TRPs with Nt=Nt'= 256.

From the figure, we make the following observations:

- For small or moderate signaling sizes, PDP samples can achieve better positioning accuracy than 2-port CIR samples at the same or smaller signaling sizes. Using PDP samples, a 90%tile 2D positioning error of 0.52 m can be achieved.

- The higher positioning accuracy achieved by 2-port CIR is only possible with very large signaling sizes.

- For most given 90%tile 2D UE positioning error requirements, the DP samples requires the smallest signaling sizes.

Figure 1 Scatter plot of feedback sizes and the achieved 90%tile 2D UE positioning errors in the {60%, 6m, 2m} InF-DH scenario for different AI/ML model input types with different combinations of lengths, sub-sampling sizes and TRP down-selection using the regular sampling grid signaling approach. The positioning accuracy is for the “small model” for centralized direct positioning trained with 40,000 samples [6].

Impact of AI/ML model location for Case 3a

For Case 3a, the location of the AI/ML model will impact whether there is a need for a new measurement to be introduced. Between the LMF and the gNB, the model output generated by the NG-RAN model can be forwarded using the legacy measurement report format (e.g., RTOA). However, depending on whether the ML model is deployed at the gNB-DU or the gNB-CU, the SRS measurement entity may or may not be the same entity hosting model inference.

- If SRS measurement entity = model inference entity, then model input does not need to be specified. This is similar to Case 1 and 2a.

- If SRS measurement entity ¹ model inference entity, the input to the model (i.e., SRS measurements) may need to be specified for F1-AP.

For Case 3a, the location of the AI/ML model (e.g., gNB-DU vs gNB-CU) impacts the need to specify new measurements for model input.

Send an LS to RAN3 to request feedback on whether the input to the AI/ML model for case 3a need to be specified.

Model output of AI/ML assisted positioning

The study of model input is commonly applicable to direct and assisted AI/ML positioning, and the same design choices can be made for model input regardless of the AI/ML approach. This is not the case for model output. While the model output is predefined (i.e., target UE location) for direct AI/ML positioning, the design choices of model output need to be considered carefully for AI/ML assisted positioning.

As discussed in section 1, we assume case 3a is an AI-extended UL-TDOA or multi RTT positioning method, while case 2a is an AI-extended DL-TDOA or multi RTT positioning method. In this section, we discuss the different aspects specific to AI/ML assisted cases (case 2a and 3a). For these cases, the following issues should be addressed:

- In order to support data collection and inference, what are the assumed model output(s)?

- What modifications to legacy measurement reporting (to the LMF) are necessary when transmitting the model output to the LMF?

Details for reporting model output of AI/ML assisted positioning

As discussed, RAN1 has agreed to support two types of model output: (a) LOS/NLOS indicator; (b) timing information. In the following, we investigate in detail how they can be specified in Rel-19.

LOS/NLOS indicator

The following was concluded during RAN1#119.

Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements

We also discussed without agreeing on whether to differentiate indication of LOS / timing reported with AI/ML from legacy reporting from previous releases.

In this section we discuss the following issues:

  - What is the meaning of the LOS/NLOS indicator when reported in case 3a

  - Should the LMF be able to differentiate whether the reported LOS/NLOS indicator is based on a model inference

Interpretation of LOS/NLOS indicator and associated timing information

In legacy positioning methods, the LOS indicator can take either a binary value (LOS, NLOS) or a soft value that provides a confidence index of the LOS estimate reported by the UE or gNB. The purpose of this LOS indicator is to inform the LMF so that measurements can be grouped according to their LOS status, and measurements which are non-line-of-sight are typically excluded from the location estimation algorithm. The LOS/NLOS indicator and the measurements (e.g., RTOA) are all about the physical, observable propagation path of the radio link.

In contrast, when AI/ML assisted positioning is used, two types of LOS paths are addressed by the AI/ML model considering the two types of model output:

- LOS/NLOS indicator. As in legacy report, this model output provides information on the likelihood of a line-of-sight propagation path, i.e., a physical LOS path. For example, for Figure 2(a), LOS/NLOS indicator='TRUE' (i.e., physical LOS path exists); for Figure 2(b), LOS/NLOS indicator='FALSE' (i.e., no physical LOS path).

  - Thus: the meaning of LOS/NLOS indicator is unchanged when AI/ML is introduced to generate it.

- Time information (e.g., RTOA). The timing information generated by the model depends on the model design. For some models, the output TOA is  the direct path between TRP and UE, regardless of LOS/NLOS status of the physical link. For example, for Figure 2(a), the timing information is for the physical LOS path (=virtual LOS path); for Figure 2(b), the timing information is for the virtual LOS path. However, these “virtual LOS” models are typically very large. Another, comparatively simpler model could produce the legacy “physical LOS” type TOA.

  - Thus: the meaning of timing information is conditioned to the model implementation when AI/ML is introduced to generate it. In legacy report, the timing information is for the observed physical path arrival. In AI/ML report, the timing information can be for the direct path (i.e., virtual LOS path).

In summary, it can be understood that the LOS/NLOS indicator addresses the physical LOS path, while time information (e.g., RTOA) may address the virtual LOS path. Here the virtual LOS path is the direct path regardless of LOS/NLOS status. In legacy DL-TDOA and DL-RTOA reports, LOS/NLOS indication is always associated with a physical TOA measurement. For AI/ML measurements, this association cannot always be assumed, and it should be possible for the TOA measurement to  be decoupled from the LOS indication.

| - Physical LoS path exist | - No physical LoS path |
| --- | --- |

Figure 2. Two types of LoS paths are addressed by the assisted AI/ML model.

Thus, when AI/ML assisted positioning is applied (Case 2a, 3a), the measurement report needs to clearly indicate to LMF whether the timing information should be interpreted in a new way, i.e., LMF should not blindly reuse the meaning in the legacy report. Conversely, if this is not clearly indicated, then the LMF may run the legacy positioning methods the same as before, i.e., exclude links that are marked with LOS/NLOS indicator = 'FALSE'. This would completely defeat the purpose of generating virtual LOS based enhanced timing information by AI/ML.

To indicate that the timing information for case 2a/3a is always based on LOS TOA, regardless of the true physical LOS state of the channel, it would be sufficient to add an indicator to the report, such as a “made with AI” flag, or a “virtual LOS TOA” flag.  Alternatively, the UE or gNB may be “forced” to produce an AI-based, virtual line of sight  measurement as demanded in the measurement request from LMF. In that case the indication is not needed.  During RAN1#118bis, a common understanding was reached that a differentiation between a Rel19 report and a Rel18 (and before) report would be enough, without the need of identifying a model output explicitly, as reflected in the agreement on the report of timing information. In this case,

- A Rel-18 (and before) timing report provides TOA for physical path, where the TOA can be generated by legacy method or AI/ML.

- A Rel-19 timing report provides TOA for virtual path, where the TOA is generated by AI/ML only.

In RAN1#119, the issue was discussed but did not converge, with one proposal updated as follow:

Proposal 3.1.5-1 For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, LMF shall be able to distinguish whether the timing information is obtained by legacy method or by Rel-19. FFS: how to ensure that LMF can distinguish

Moreover, RAN3 also discussed the interpretation with the following understanding captured in the chair’s notes at RAN3#126:

RAN3 agreed that the mechanisms and IEs can be reused: -Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. ›One open point on the procedures for the LMF to request positioning data for case 3a is the following: FFS if LMF should be aware that the reported measurements are AI/ML generated.

Hence, for RAN3 to progress, RAN1 should conclude on whether LMF need some awareness to distinguish Rel-19 timing measurement from legacy.  Our view is that Proposal 3.1.5-1 from RAN1#119 can be updated to addresses the issue, and allows to support Case 3a where depending on the model design choice, the model output can be both the timing information for a virtual LOS path, but can also be backward compatible with the timing info of Rel18 physical LOS path. We propose to clarify what is implied by Rel-19 report, which is that the timing information and LOS information are decoupled, and that timing information is reported for virtual LOS.

The LOS/NLOS indicator in legacy positioning report is a measure of the reliability of the measurement report.

AI/ML assisted positioning can provide measurement report of the NLOS channel with high confidence.

For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, the timing information is for virtual LOS link if carried in a new Rel-19 measurement report, and the timing information is for physical LOS link if carried in a legacy measurement report (Rel-18 or prior).

For Rel-19 measurement report, the LOS/NLOS indicator (for physical LOS link) is decoupled from the timing information reporting (for virtual LOS link).

Support reporting (optionally) a LOS/NLOS indicator when timing information is reported for Rel-19 AI/ML positioning Case 3a.

Reporting of a of LOS/NLOS indicator based on model output

As explained in section 4.1.1.1, the LOS information in a case 3a report must be decoupled from a timing measurement. When generated by the AI/ML model, the timing measurement is always a “LOS” measurement, that is to say a time-of-flight measurement, so that the corresponding indicator would always be a “LOS”, and thus be completely meaningless to report. A decoupled LOS estimate, reflecting the physical nature of the channel, would however convey useful information.

The definition of the LOS indicator should in our view be left unchanged, and reflect the physical nature of the channel as in Rel-18 and prior. What remains to discuss is whether the LOS indicator should be identified based on how it was derived, e.g. based on a model output or pre-Rel19 methods (which does not say anything on how the indicator was computed).  RAN3 has also concluded in the same direction during the November 2024 WG meeting:

RAN3#126: ›Concerning the IEs in the existing measurement request procedure, the following was captured reflecting the common understanding: -Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used.

If RAN1 wants to support the LCM for a model generating LOS NLOS beyond self-monitoring, there may be a need for identifying the LOS indicator as model output, so that e.g. the LMF may feedback to the gNB that the LOS estimate is not correct based on other information the LMF may have. Otherwise, the motivation for identifying the LOS output (legacy vs AI-generated) is not clear, since the information carried by the indicator is not changed from pre-Rel-19.

The interpretation of a LOS indicator as reflecting the physical nature of the channel should be the same in Release 19 and legacy.

If the LMF is not involved in the model monitoring of the model generating the LOS indicator, the need for identifying a LOS indicator as generated by a model output is not clear.

Summary of LOS/NLOS indicator reporting

In Table 7, the possible combinations of timing measurement and LOS/NLOS indicator are listed, together with their support, if applicable.

Table 7. The possible combinations of timing measurement and LOS/NLOS indicator

|  |  | Associated measurement (e.g., timing) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Absent | Legacy measurement* | Timing measurement generated by AI/ML |
| LOS/NLOS indicator | Absent | N/A | Support (Existing specification) | Support in Rel-19 |
|  | Generated by legacy methods | Not supported (Existing specification) | Support (Existing specification) | Support in Rel-19 |
|  | Generated by AI/ML | Not supported in Rel-19 | Support in Rel-19 | Support in Rel-19 |

* When the associated legacy measurement is reported, it is not limited to timing measurements. It can be timing measurement or angle measurement.

For reference, in TS 38.455, LOS/NLOS indicator is reported per TRP, with one indicator for a given ULRTOA measurement (containing one or more paths) defined as follow: soft values provide the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Hard value '0' indicates NLOS and hard value '1' indicates LOS.

In summary, we see LOS/NLOS indication in a Case 3a report as follow:

- The definition of LOS/NLOS indicator is the same as legacy. It is unnecessary and confusing to introduce new meaning of LOS/NLOS indicator, depending on which report format is used.

- For Case 3a, the LOS/NLOS indicator, when reported with a Rel-19 ULRTOA measurement identified as such (as agreed in RAN1#118bis), should be understood as decoupled from the timing measurement.

- The LOS/NLOS indicator can optionally be reported, the same as in legacy.

- The indicator does not need to be made a separate measurement, the same as in legacy.

Thus we propose the following revised proposals on LOS/NLOS indicator for Case 3a.

For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, from RAN1 perspective, the LOS/NLOS indicator provides information on the likelihood of a physical Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LOS and a value of 0 corresponding to NLOS; the LOS/NLOS indicator reuse the same format as the existing IE "LOS/NLOS Information" in 38.455.

(for conclusion) For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, do not support an indication of whether the LOS/NLOS indicator is obtained via a model output or via legacy method.

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, one LOS/NLOS indicator is optionally reported per TRP in a measurement report from gNB to LMF, as in the existing specification.

For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, LMF shall be able to distinguish whether the timing information is obtained for Rel-19 measurement report (virtual LOS link) or for legacy measurement report (physical LOS link).

- For timing information obtained for Rel-19 measurement report, If a timing measurement is reported together with the LOS/NLOS indicator, the indicator status (for physical LOS/NLOS) is decoupled from the reported timing measurement (for virtual LOS)

- Note: when the TRP-UE radio link is LOS, the virtual LOS coincides with the physical LOS.

- Details of how to associate the Rel-19 timing information with virtual LOS is up to RAN2/3, e.g. indication in measurement report, or configuration in measurement request.

Timing information

For AI/ML assisted positioning, RAN1 has agreed that the reported timing information are reported in the format of UL-RTOA, DL-RSTD, gNB RxTxTimeDiff and UE RxTxTimeDiff. Thus, the existing measurement reporting in LPP and NRPPa can be easily reused.

Reuse the existing measurement reporting in LPP and NRPPa to report timing information for the model output of AI/ML assisted positioning.

For Case 3a, after model output (UL-RTOA) is generated, post processing can be applied to the model output to generate the timing information for measurement report, where the reported timing information can follow any desired format according to the associated legacy positioning method. This is illustrated in Figure 3.

Figure 3. Illustration of postprocessing AI/ML assisted model output to provide measurements for reporting to LMF.

As illustrated in Figure 3, thanks to the postprocessing of model output, only one model needs to be trained and maintained for Case 3a, regardless of the TRP measurement result to report. This has several benefits compared to having different models for different measurement results (i.e., UL RTOA, gNB RxTxTimeDiff):

- For all LCM stages including training data collection, model training and model monitoring, the complexity and overhead are much lower when only one model is needed instead of two;

- For model inference, using one model instead of two saves chip area, memory requirement, cost, and power consumption.

- For model inference, the LMF can freely make a request for UL RTOA or gNB RxTxTimeDiff without considering that gNB may need an extra delay to switch models, if the gNB needs to activate a different model according to the measurement request.

Based on the discussion above, the following are proposed.

For AI/ML assisted positioning at gNB (Case 3a), the model output is uplink relative time of arrival (TUL-RTOA).

For AI/ML assisted positioning at gNB (Case 3a), postprocessing is applied to generate gNB RxTxTimeDiff for measurement reporting of multi-RTT method.

Indication of AI/ML assistance in generating measurement reports

Based on the previous section, it is clear that there is a need to differentiate measurements generated with AI/ML model assistance from legacy measurements, since the actual interpretation of the timing measurement differs.

To identify the meaning of the timing information sent to LMF, and more generally what part of the UE/gNB report is produced with assistance from the AI/ML model, we see three possible options:

- Option 1: A new report message specific to AI/ML assisted positioning cases 3a/2a

- Option 2: an indicator in the existing report, indicating that the report was produced using AI/ML assistance.

- Option 3: an indicator is attached to the elements of the reports where AI/ML was used to generate it.

All options could be discussed by RAN3/RAN2 in terms of signaling optimization, but RAN1 is responsible for answering whether the LMF needs to know exactly what part of the report was using AIML.  In our understanding, Case 3a and Case 2a measurement reports will consist of measurements only produced by AI/ML. For example, line of sight indication, UL-RTOA, DL RSTD measurement will be based on the model output. Hence from our perspective, it is sufficient to indicate to the LMF that the measurement report as a whole is based on AI/ML model output, without having to tag each quantity in the report (e.g. LOS indication, RSTD measurement, additional measurement, etc) with a “made with AI” indicator.

For AI/ML assisted positioning at gNB (Case 3a) and UE (Case 2a), the measurement report contains an indicator that AI/ML is used to produce the measurements. The signaling details are up to RAN3/RAN2.

Training data collection

Content of training data samples

For the data samples to be collected for training, the following agreement was made in RAN1#116bis.

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement

For supervised learning, ground truth label needs to be collected for each entry of training data. On the other hand, it may be difficult to obtain a large number of accurate ground truth labels. When ground truth label is not available, the measurement data (corresponding to model input) can still be collected and used by semi-supervised learning. Thus both labelled and un-labelled training data samples can be collected.

For Rel-19 AI/ML based positioning, support collecting labelled and optionally un-labelled training data samples.

In the following sub-sections, details on Part A and Part B are further discussed.

Time stamp of training data samples

In RAN1#118, the following agreement was made on the time stamp of channel measurement:

Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

In RAN2#127, the following agreement was made on NW side data collection:

Agreements - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network.

As can be seen in RAN2 agreement above, the baseline procedure is that the UE stores training data in its memory, and report a batch of logged data at a time. That is, the training data reporting time and measurement time are separate, and can be far apart, i.e., likely the reporting time is far later than the SFN wrap-around time of 10240 ms (=10.24 sec). Therefore, one cannot rely on an entity that receives the training data to add time stamp; rather the measurement entity itself should add time stamp as the training data is being collected.

While the RAN2 agreement above is on UE procedure, it is reasonable to expect that the same principle applies when the training data generation entity is on the NW-side (i.e., gNB or LMF for positioning). RAN3 specifications support the use of periodicity of up to 30 minutes for reporting of measurements. In that case, SFN/slot level time stamps become ambiguous and a measurement time reference is necessary.

To clear the ambiguity, the report should always include measurement time whenever the reporting periodicity is too large for the reported measurement to be unambiguously mapped to a time instant using only SFN/slot numbers. This can be handled by RAN2 and RAN3 groups, but RAN1 should at least agree on the need for unambiguous mapping between part A and part B.

Thus the time stamp for both Part A and Part B should be mandatorily added by the training data generation entity, and the time stamp should use a much longer range than SFN. Specifically,

- for channel measurement and label generated by PRU or non-PRU UE, always include the existing IE UTCTime (e.g., for utc-time-r16) in TS 37.355 (i.e., not a choice between SNF and UTCTime);

- for channel measurement generated by TRP/gNB, either change the field “Measurement time” in TS 38.455 from optional to mandatory, or specify that the field “Measurement time” is present whenever the measurement reporting periodicity exceeds 10.24 sec.

Regarding the time stamp resolution, several different granularities exist in current specifications, for example:

  - LPP:

    - UTCTime with a granularity of seconds;

    - utcTime-ms with a granularity of milliseconds;

  - “Measurement time” with a granularity of 1 /2**32 second.

In our view, there is no need to align to a same granularity for time stamp of Part A and Part B, and existing IEs can be reused. Regarding how close the stamped time should be between Part A and B so that they can be paired, this is likely related to the positioning accuracy requirement, UE speed, and also related to the implementation of training data collection entity and model training entity. From RAN1 perspective, there is no need to impose any restriction on how to perform the pairing using the time stamps, including the allowed time difference between Part A/B so that they can be paired.

For training data collection of AI/ML based positioning, for both Part A and Part B, time stamp is mandatorily provided by the training data generation entity.

For training data collection of AI/ML based positioning, the time stamp should allow unambiguous mapping between Part A and Part B.

  - For channel measurement generated by TRP/gNB, at least when measurement periodicity exceeds the SFN/slot wrap around time (10.24 sec), field “Measurement time” is included in the time stamp.

  - For channel measurement and label generated by PRU or non-PRU UE, UTC Time (e.g., for utc-time-r16) is included in the time stamp.

  - Exactly how to specify time stamp for Part A and Part B is up to RAN2/RAN3.

From RAN1 perspective, the existing IEs and their associated granularities can be reused for time stamp of Part A and Part B. It is acceptable to have different time stamp granularity for Part A and Part B. There is no need for RAN1 to discuss how to perform the pairing using the time stamps.

For reference, existing signaling in 37.355 (LPP):

| Terrestrial Beacon System Positioning | TBS-MeasurementInformation-r13 ::= SEQUENCE { measurementReferenceTime-r13 UTCTime OPTIONAL, mbs-SgnMeasList-r13 MBS-BeaconMeasList-r13 OPTIONAL, -- Cond MBS … } measurementReferenceTime This field provides the UTC time when the TBS measurements are performed and should take the form of YYMMDDhhmmssZ.. |
| --- | --- |
| NR DL-TDOA Positioning | NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, … } OPTIONAL, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |
| NR DL-AoD Positioning | NR-DL-AoD-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { sfn-time-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |

UTC-Time-r15 ::= SEQUENCE {

utcTime-r15					UTCTime,

utcTime-ms-r15				INTEGER (0..999),

...

}

utcTime This field provides the time stamp of the refTime in UTC time and comprises the following subfields: - utcTime in the form of YYMMDDhhmmssZ. - utcTime-ms specifies the fractional part of the UTC time in ms resolution.

For reference, existing signaling in 38.455 (NRPPa):

9.2.42		Time Stamp

This information element contains the time stamp.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  |
| CHOICE Slot Index | M |  |  |  |
| >SCS-15 |  |  | INTEGER(0..9) |  |
| >SCS-30 |  |  | INTEGER(0..19) |  |
| >SCS-60 |  |  | INTEGER(0..39) |  |
| >SCS-120 |  |  | INTEGER(0..79) |  |
| Measurement time | O |  | Relative Time 1900 9.2.36 |  |

9.2.36	Relative Time 1900

This information element indicates the initialisation time (e.g. SFN Initalisation Time for a cell, requested time for an action, etc).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Relative Time 1900 | M |  | BIT STRING (SIZE(64)) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |

Signalling and quality reporting for Part A

From the RAN 1 perspective, data collection for Part A is the same procedure as measurement reporting /requests for inference. Additionally, legacy measurement request may be re-used.

For Case 1 and Case 2a, data collection for Part A between a UE and another PRU UEs is already supported by specification. For data collection between non-PRU UEs, 3GPP should not specify a method for data collection. Rather, an OTT solution is assumed.

From the RAN1 perspective, current specification support for Part A data collection in Case 1 is sufficient.

For Case 3b and 2b, data collection for Part A should not be different from measurement request during inference. For Case 3a with model in the gNB (CU if split architecture) the situation is the same.

From the RAN1 perspective, Part A data collection in Case 2b and 3b is identical to measurement reporting for inference.

For Case 3a with model in the TRP, the TRP can handle its own measurements. Thus measurement request for Part A in Case 3a is not an issue.

From the RAN1 perspective, Part A data collection in Case 3a is either up to the network implementation or identical to measurement reporting for inference.

Based on this analysis, RAN1 does not need further discussion on specific procedure for Part A data collection.

RAN1 does not need to discuss further signalling to enable Part A data collection.  Measurement reporting is jointly discussed for training data collection and inference.

For quality indication of model input, i.e. Part A, the measurement quality can also be jointly discussed for training data collection and inference, since the same measurement quality indicator type is expected to be used for measurement reported during inference.

Regarding timing measurements in Part A, e.g., the sample timing in PDP and DP, it was agreed in RAN1#118bis that the existing IE for timing quality is reused, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455.

Regarding power measurements in Part A for PDP, quality indicator similar to NR-TimingQuality can be provided. An example is shown below.

NR-PowerQuality-r19 ::= SEQUENCE {

powerQualityValue-r19			INTEGER (0..9),

powerQualityResolution-r19		ENUMERATED {dBdot1, dB1,…},

...

}

NR-PowerQuality field descriptions

powerQualityValue This field provides an estimate of uncertainty ±X dB of the power value for which the IE NR-PowerQuality is provided in units of dB. Represents the deviation (uncertainty) in power as compared to what is reported for power level. Scale factor is provided in powerQualityResolution.

powerQualityResolution This field provides the resolution used in the powerQualityValue field. Enumerated values dBdot1, dB1 correspond to 0.1, 1 dB, respectively.

The power quality indicator is provided for the entire PDP measurement, i.e., not for individual sample/path of the PDP. This is because all the sample/path are for the same channel measurement, and are detected by the same receiver.

For training data collection in all cases, a power quality is provided for the entire PDP measurement in Part A in units of dB. The candidate resolution includes 0.1 dB and 1 dB.

For reference, existing signaling in 37.355 (LPP):

–	NR-TimingQuality

The IE NR-TimingQuality defines the quality of a timing value (e.g., of a TOA measurement).

-- ASN1START

NR-TimingQuality-r16 ::= SEQUENCE {

timingQualityValue-r16			INTEGER (0..31),

timingQualityResolution-r16		ENUMERATED {mdot1, m1, m10, m30, …},

…

}

-- ASN1STOP

NR-TimingQuality field descriptions

timingQualityValue This field provides an estimate of uncertainty of the timing value for which the IE NR-TimingQuality is provided in units of metres.

timingQualityResolution This field provides the resolution used in the timingQualityValue field. Enumerated values mdot1, m1, m10, m30 correspond to 0.1, 1, 10, 30 metres, respectively.

Signalling and quality reporting for Part B

For training data collection of Part B, the label generation entities for each case is already agreed by RAN1.  The working assumptions could be confirmed in order to close the discussion.

Confirm the existing WAs on label generation for all cases and send the agreements to RAN2.

Regarding ground truth label in Part B, in our view, the training dataset is agnostic of the model architecture and model training methodology. To reduce the burden of collecting, storing, and updating training datasets, a same training dataset should be usable to train a variety of AI/ML models for positioning.

- For DL measurement, it should be possible to use the same collected training dataset to train a model for Case 1, and/or 2a, and/or 2b.

- For UL measurement, it should be possible to use the same collected training dataset to train a model for Case 3a and/or 3b.

- For AI/ML assisted positioning, it should be possible to use the same collected training dataset to train a model that produces {timing measurement} as output, or produces {timing measurement, LOS/NLOS indicator} as output.

The details of whether/how to train a range of models using a given training dataset is up to each vendors’ design choice.

Considering the above, for all the positioning cases, the ground truth label should be collected in the format of UE/PRU location coordinates (x, y, z). This has the benefit of convenient data collection (as compared to ground truth for RTOA or LOS/NLOS indicator, for example). This also makes the training dataset versatile and generally applicable to all positioning cases.

Then, depending on the design choice of the model, the collected label can be further processed either before model training or during model training. The details are up to implementation. For example, if it is desired to train a Case 3a (or 2a) model with RTOA as model output, then the RTOA ground truth can be calculated from the collected UE/PRU location coordinates (x, y, z), based on the knowledge of TRP locations. Similarly, if it is desired to train a Case 3a (or 2a) model with LOS/NLOS indicator as model output, then the model training procedure can be designed to use coordinates (x, y, z) as proxy labels.

In the most straight-forward manner, for assisted AI/ML positioning (Case 2a/3a), training data collection directly collects ground-truth labels on radio measurements, which can be difficult, costly, and even impossible to obtain. It is possible to circumvent those issues by using "ground truth" position data as indirect labels, rather than directly labelling radio measurements.

If the estimated UE position p is obtained by minimizing a loss function fθp with respect to p, assuming a vector of neural network outputs θ=θ1⋯θm∈ℝm containing e.g., TOAs, soft LOS indicators, etc., then the derivative of p with respect to θj is:

∂p∂θk=−Hpfθp−1∇p∂fθ∂θkp

where Hpfθp is the Hessian with respect to p at p. This expression can be used in training to back propagate over this optimization step using a ground truth label for the UE position. By leveraging real-world location data as a proxy, this method enables the model to infer labels on radio measurements (such as soft LOS indicators, etc) indirectly during training.

This technique involves back-propagation across both trainable and untrainable optimization steps, allowing the model’s outputs to align with the desired positioning estimates without needing direct measurement labels. By substituting position data for radio measurement labels, the approach enhances the flexibility and scalability of model training. This indirect labelling strategy not only simplifies the data collection process but also improves the model's adaptability across different network environments, facilitating accurate and efficient positioning.

For training data collection Part B, support only ground truth labels in the format of UE location coordinates (x, y, z).

For label quality, RAN1 extensively discussed how to attach a quality to part B, with the following being the latest proposal from RAN1#118 bis:

Proposal 4.1.5-1A (RAN1#118bis) For training data collection of AI/ML based positioning Cases 1, 3a, 3b, from RAN1 perspective, when label reporting is applicable, quality indicator of label is represented using one of the following options. RAN1 study and select between Option 1 and Option 2. - Option 1: reuse an existing IE. For each case, proponent companies provide input on the existing IE to use. - Option 2: define a new IE on UE location quality. Use hard (0 or 1) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicating the highest quality. - Proponent companies provide information on how to determine the value, entity to generate the label quality, and the applicable signaling.

Option 1 can already be supported by a UE or LMF for positioning solutions prior to Rel-19.  Note that when providing a location information, a UE also reports the method used to generate that information (e.g. GNSS, WLAN, DL-TDOA).

Option 2 provides a generic metric for quality. However, the quality level depends also on the specific positioning use case,  the desired positioning accuracy, and the AI/ML model design (e.g., structure, size, parameter quantization, etc). Such information may not be known at the time the training data is collected. Furthermore, a given training data set can be used to train multiple models, potentially each with a different accuracy target and a different model design. Depending on the training context, as an example, some deployment/model may treat 1m accuracy as low quality, while another treat 1m accuracy as high quality. Also, Option 2 requires high standardization effort to define the quality values, e.g., how to determine the quality value given a UE location error in meters.

During RAN1#119, further discussion on the options clarified that the purpose of option 2 is for the LMF to provide a measurement quality which is computed by the LMF.  The proposal was reworded as follows by a proponent company:

Proposal 4.1.4-2 For AI/ML based positioning, when label is UE location and reported (e.g., LMF to UE over LPP), label and quality indicator of label are provided by the following options: reusing existing IE. - Reusing existing IE, the existing IE can use one of the geographic shapes defined in TS 23.032. The uncertainty and confidence serve as quality indicator of the label. - Using a new IE on UE location quality supported by LMF. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. Note: Label quality indicator may be considered as an additional UE capability.

Our understanding is that the part B label will be signalled re-using the signalling for providing location, i.e. a new “label IE” will not be introduced. Since there is already a measure of quality of the location in the measurement uncertainty and confidence, we do not see how an additional quality indicator will provide further information to the UE. The existing indicator can already convey the LMF assessment of the location quality.

Regarding the specific quality metrics, the existing IE LocationUncertainty has been defined to provide horizontal and vertical information for ARP, as shown below from LPP. Similar IEs exist for NRPPa. Label uncertainty is analogous to ARP location in that these location information are considered known, though with error margins in real-world applications. This is in contrast to location of target UE, which is unknown, and to be determined using the positioning methods.

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r16				INTEGER (0..255),

horizontalConfidence-r16				INTEGER (0..100),

verticalUncertainty-r16					INTEGER (0..255),

verticalConfidence-r16					INTEGER (0..100)

}

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The ‘verticalUncertainty’ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ‘verticalConfidence’ corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.

For horizontal and vertical uncertainty, the value range of (0..255) indicate a location error range between 0 and 46.49129 metres, with the smallest indicated value of 6 millimeters, see Appendix A. In our view, the existing horizontalUncertainty-r16 and verticalUncertainty-r16, and their corresponding confidence, can be reused to indicate the UE location quality for labels, including the encoded high accuracy uncertainty as defined in TS 23.032. The definition of the fields would naturally clarify that the uncertainty values are for UE location (PRU or non-PRU UE), instead of ARP.

Additionally, one question is whether the label quality should be provided differently depending on model output, for example, timing information (UL RTOA) for assisted positioning or UE location for direct positioning. As explained earlier, in our view, the training dataset is agnostic of the model architecture and model training methodology. Thus, for the purpose of training data collection, label quality only needs to be recorded as UE location error in meters. If the model training entity has a need of a quality of an intermediate metric (e.g., UL RTOA), then such quality can be calculated using the UE location error in meters. Such procedure is out of scope for RAN1 standardization in our view.

Hence, keeping legacy quality metrics is preferable. We support Option 1, and don’t support Option 2.  The following proposal discussed in RAN1#119 can be revised and supported.

Proposal 4.1.4-2 (RAN1#119) For AI/ML based positioning, when label is UE location and reported (e.g., LMF to UE over LPP), label and quality indicator of label are provided by reusing existing IE. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The uncertainty and confidence serve as quality indicator of the label.

From RAN1 perspective, the same label (UE location) and label quality (UE location uncertainty) are collected, irrespective of the type of model architecture (e.g., assisted or direct positioning) that may be selected by the model training entity.

For training data collection, the label quality for Part B reuse the existing IEs for indicating UE location quality. From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The uncertainty and confidence serve as quality indicator of the label.

Pairing of Part A and Part B in data collection

During RAN1#119, several companies brought forward the issue of pairing part A and part B. Correspondingly, Proposals were drafted to discuss this issue in RAN1#119:

Proposal 4.4.2-1 For training data collection of AI/ML based positioning, if Part A and Part B are generated by the same entity, the paired Part A and Part B are provided in one report. - If multiple entries of Part A and Part B are reported, the Part A entry and the corresponding Part B entry are grouped in a pair for reporting. - The paired Part A and Part B entry share the same time stamp. Proposal 4.4.2-2 For training data collection of AI/ML based positioning, if Part A and Part B are generated by different entities, the pairing between a Part A entry and a Part B entry is supported via at least: - A UE identifier, and - The time stamp of Part A and the time stamp of Part B. - Note: Different time stamp granularity can be used for Part A and Part B. For example, “Measurement time” in 38.455 and UTCTime in 37.355 have different granularities.

Some companies questioned whether RAN1 is the right WG for the issue, or if there is actually an issue. In our view,  it is paramount that for both the case where part A and part B are generated by the same entity, or separate entity, the node receiving part A and / or part B must be able to relate them. Even if a time stamp is not exactly the same, the pairing may be possible, e.g. when a UE is static for a long time. Moreover, when the UE is either static or slowly moving, it is not efficient for labels to be recorded for every measurement. Instead, a single label can be applied for multiple time stamps, or a span of time stamps.  The issue is relevant to all cases, since all involve label collection in some way.

Time stamp for Part B may span a period of time for which the location of the UE is consistent.

Part A-B pairing is an issue across Case 1, 3a and 3b.

A label provided by a LMF, PRU, or non-PRU UE can be attached with one or more time stamp, or a span of time stamps, for which the label is valid.

The second issue for part A and part B pairing is knowledge of the UE involved in the collected measurement and label, e.g. securing that the same UE/PRU is used during the collection training measurement data in part A and the label in part B.  such issue involves securing the  UE ID and, when needed, UE consent. It is thus better suited for RAN2/RAN3/SA to resolve this issue.

Part A and Part B must be paired for the same UE, even though Part A and Part B may be reported by different entities.

The issue is non-trivial at least  in Case 3a when the UE is not measured by its serving cell, which already knows the UE local identity since it has configured the SRS to be transmitted and measured on. A non-serving cell is only performing measurement but does not necessarily know the UE. It is therefore unclear how part B and part A can be paired in that case.

Send an LS to RAN2 and RAN3 requesting to investigate how Part A and Part B can be matched to the same UE for Case 1/3a/3b.

Metadata for training dataset

Elements in metadata of the training dataset

To keep consistency between training and inference phase, it is crucial that the node operating the model can secure that the assistance data and signals received during inference are the same as what was used for training the model. For this purpose, in addition to the list of training data samples, the training dataset also needs to record the context information of the collected data. Such context information is attached to the training dataset as its metadata. The context information should include basic information like:

- Where the training data is collected

- When the training data is collected, for example, year/month/date

- Which nodes provided the measurement data, for example, TRP ID, UE ID.

- Which nodes provided the label data, for example, PRU ID, UE ID, LMF ID.

- What kind of reference signals are used in generating the measurement data, for example, PRS configuration for DL measurement, SRS configuration for UL measurement.

- Type and format of the measurement data, for example,

  - Type (e.g., DP/PDP) and size (e.g., Nt, N't) of the measurement data

  - Quantization granularity used in recording the measurement values

- Type and format of the label data, for example,

  - Type of information for label data, for example, UE location or information corresponding to model output of AI/ML assisted positioning models;

  - Format used to record the location information (for labels),

  - 2D or 3D location coordinates

- Criteria (if any) applied before a data sample can be accepted as part of the training dataset, for example, label quality requirement, SINR requirement, etc.

- Version ID of the training dataset, if the training dataset may be updated over time.

Regarding the criteria applied to collect data samples of adequate quality, it should cover both measurement data (corresponding to model input) and label data (corresponding to model output). This gives the high-level information on the training dataset:

- The accepted range of measurement data quality.

- The accepted range of label error (or: uncertainty of ground truth label).

When the training dataset is used for training a model, the model training entity may further decide to include only data samples with sufficient quality, where the quality is determined according to the targeted positioning accuracy requirement. For instance, during training data collection, data samples with label error < 2.0 meters are collected into the training dataset. During model training, two models may be trained, one model is trained using filtered data samples with label error < 1.0 m for higher accuracy applications, while the other using all data samples (thus label error < 2.0 m) for lower accuracy applications.

Specific to training data collection of Part A, the model host (TRP, gNB, LMF or UE) will integrate measurements together with related metadata. To enable accurate inference result, the metadata should ideally not differ between the training data set and the measurement performed during inference.  The metadata that need to be recorded for Part A of the training dataset can be grouped as follows:

- Group 1, assistance data.  In the positioning framework, assistance data is sent from LMF to either gNBs or UEs to enable reference signal measurements. For training data collection, the LMF should provide suitable assistance data to the measuring nodes.

- Group 2, signal measurement configuration, for example, PDP or DP, measurement parameters {Nt, Nt', k}. For Case 2b, 3b, and potentially 3a, this is the information included the measurement request, sent from LMF to UE or gNB, for model inference. For cases where the channel measurement is not sent to LMF for model inference (Case 1, 2a, and potentially 3a), the analogous signal measurement information should be recorded as well.

- Group 3, measurement environment condition and quality. This may include: network synchronization error range, UE/gNB RX and TX timing error range, interference range, channel estimation error range, etc.

Thus RAN1 should discuss for each of these groups firstly whether new signalling is needed, and second how to keep consistency between training and inference.

For Group 3, Rel-18 study item evaluations show that such context information also affect the validity of the trained model in the deployment scenario. Thus such context information about the training dataset also needs to be recorded as part of the metadata, including:

        - Range of SNR/SINR

        - Range of Channel estimation error

        - Range of NW synchronization error

        - Range of UE/gNB RX and TX timing error

        - …

The training dataset is attached with metadata which describes the context information on the set of collected training data samples.

For training data collection of Part A, the metadata includes: Group 1 for assistance data, Group 2 for signal measurement configuration, and Group 3 for measurement environment condition and quality.

RAN1 to study the details of Group 3 metadata to attach to training dataset, including:

- Range of SNR/SINR

- Range of Channel estimation error

- Range of NW synchronization error

- Range of UE/gNB RX and TX timing error

The metadata of the training dataset is not only necessary for keeping track of the context and versions of the training dataset, it also provides context information of model(s) trained with the given training dataset. During model deployment, the context information of the model is needed to ensure consistency between model training and model inference. Further details are provided in section 7.1 and 7.2.

Furthermore, the AI/ML solution needs to secure a training dataset of adequate quality as a whole.

- Collect a sufficiently large size of training dataset.

- Collect data that fully reflect the UE distribution during operation, e.g., uniformly distributed over the entire floor.

- Collect data from a variety of UEs to generalize over different UE implementations, including different UE vendors, different UE capabilities, different UE orientation, etc.

While the training data collection procedure needs to pay attention to the above issues, they are expected to be taken care of via engineering effort and not explicit in the standard specification.

Collecting sufficient data in terms of training dataset size, UE distribution, diversity of UE sources, etc, is up to implementation.

Assistance data for consistency between training and inference

Once the assistance data for data collection is established, the AI/ML positioning framework should also enable consistency between assistance data received during data collection and inference.  In our view, such consistency can be achieved to a large degree using a validity area attached to the training data set. Considering the model generality issues studied in Rel-18 SI, finger-printing based AI/ML methods are very sensitive to the deployment area. A model trained for one deployment area is unlikely to work for a different deployment area without any change. During RAN1#117, it was discussed whether a training data set validity area should be explicitly attached to a training data set, i.e via a validity area ID, or if the training data set should contain information sufficient for a node receiving the training data set to determine the validity area of the model being trained and leave the consistency check to implementation.  In our view, explicit definition of the model validity area is needed.

We see the need of validity area for the following reason:

- Consistency of assistance data between data collection and inference. In case 1 or 2a, the UE perform data collection with a given set of assistance data and train its model, then receive assistance data at the inference phase. The AD for inference must at least overlap with the AD used during data collection to train the model. The UE could of course get a set of AD and request what AD is missing via on-demand PRS request, but this is inefficient signalling, since it is possible for the LMF and the UE to handshake which AD is needed during inference thanks to an agreed upon validity area established during data collection, if a validity area ID is defined.

- For the LMF, knowledge of the validity area of a model can also be used to decide whether the model is compatible with the AD it intends to send. For example, the network may have changed the PRS configuration between data collection and inference, making the UE untrained for the new deployment. From the overhead perspective, it is more efficient for the network to know in advance if the AD is useful and thus know that starting the procedure for AI use cases is worth it, or if legacy positioning should be used instead. Hence knowledge of validity area is also valuable at the LMF side.

Hence, an ID associated with the UE’s model training dataset validity area can reduce the amount of overhead to align assistance data between training and inference.

Thus, for the training data collection stage, it is necessary to define a training dataset validity area Adataset. This describes the area that training dataset is collected, and can be defined as:

| Training dataset validity area | The network area for which the training dataset is valid. |
| --- | --- |

Similar to the existing 'AreaID-CellList', the validity area for AI/ML operation can be defined as a list of TRPs, where the TRPs transmits DL PRS and covers the surrounding area to support positioning. Checking the indication of the TRP ID in TS 37.355, it includes dl-PRS-ID-r16 in addition to the fields in NR-Cell-IDs-r17.  This shows that an IE can be easily defined to provide area information using a list of TRP IDs.

TS 37.355:

– AreaID-CellList The IE AreaID-CellList provides the NR Cell-IDs of the TRPs belonging to a particular network area where the associated assistance data are valid. Each cell is included in only one area. -- ASN1START AreaID-CellList-r17 ::= SEQUENCE (SIZE(1..maxCellIDsPerArea-r17)) OF NR-Cell-IDs-r17 NR-Cell-IDs-r17 ::= SEQUENCE { nr-CellGlobalID-r17 NCGI-r15 OPTIONAL, -- Need ON nr-PhysCellID-r17 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-ARFCN-r17 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON ... } -- ASN1STOP

TRP-LocationInfoElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON associated-DL-PRS-ID-r16 INTEGER (0..255) OPTIONAL, -- Need OP trp-Location-r16 RelativeLocation-r16 OPTIONAL, -- Need OP … }

Therefore, the training dataset validity area can be defined using the following:

– AreaID-TRPList The IE AreaID-TRPList provides the NR TRP IDs of the TRPs belonging to a particular network area. For AI/ML training data collection, AreaID-TRPList provides the network area where the training data is collected. For a trained AI/ML model, AreaID-TRPList provides the network area where the AI/ML model is valid. A TRP ID can be included in different network areas described by AreaID-TRPList. -- ASN1START AreaID-TRPList-r19 ::= SEQUENCE (SIZE(1..maxTRPIDsPerArea-r19)) OF NR-TRP-IDs-r19 NR-TRP-IDs-r19 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-CellGlobalID-r17 NCGI-r15 OPTIONAL, -- Need ON nr-PhysCellID-r17 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-ARFCN-r17 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON ... } -- ASN1STOP

The training dataset validity area can be recorded as a part of the metadata for the training dataset, where the metadata describes the model training context. With the list of TRPs recorded as a part of the meta data for the training dataset, the measurements collected for each data sample can be organized with the same hierarchy as for legacy assistance data, i.e. per TRP/frequency layers for the UE, and per TRP for the gNB assistance data.

The training dataset validity area is recorded as a part of metadata of the training dataset. The validity area is defined as a list of TRPs where data is collected.

Training data provided to UE/gNB is organized with the same hierarchy as for legacy assistance data, i.e. per TRP/frequency layers for the UE, and per TRP for the gNB assistance data.

If the UE collects measurement data based on PRS from a list of TRPs, then the UE record the associated TRP IDs as the validity area of the training data.  Assistance data on the PRS configuration, PRS index used in measurement, etc, are also recorded as metadata of training dataset as well.

Furthermore, while the validity area and the RS assistance data are very similar to the assistance data for legacy positioning methods, a new assistance data needs to be introduced for associated ID. For AI/ML based positioning, the associated ID indicates the additional network conditions, and provides information on the consistency between training data collection and model inference, see related discussion in section 7.2.3.

In summary, for Case 1/2a, the assistance data for training data collection includes:

  - Training dataset validity area,

  - The assistance data necessary for PRS reception. In general, the assistance data that can be provided from LMF to UE (info #1-15, except info #7) as agreed in RAN1#119 is more than enough.

  - Associated ID to indicate the additional network conditions. The additional network conditions are those that are not explicitly signaled but may affect the consistency between training data collection and model inference, for example, info #7. Details of associated ID are discussed further in section 7.1.

For Part A training data collection of Case 1/2a, the assistance data include: training dataset validity area, assistance data necessary for PRS reception, and associated ID for additional network conditions. Corresponding assistance data is sent during model inference as well for consistency checking.

The assistance data required for PRS/SRS reception is the same across all use cases for both direct and AI/ML assisted positioning.

LCM and Model performance monitoring

Model-level and functionality-level monitoring

During RAN1#118, we discussed which entity (LMF, gNB, TRP, UE) should make LCM decision, particularly whether entity beside the model hosting entity should make LCM decision for the model hosting entity:

Proposal 6.2.5-1 For label-based model performance monitoring of AI/ML positioning Case 1, study the following options: - Option 1: the target UE makes the functionality/model management decision/recommendation (e.g., activate or deactivate a model, update a model, fallback to legacy positioning methods) as a result of model performance monitoring. - The target UE notifies the LMF of its functionality/model management decision/recommendation. - Option 1 is at least applicable if Option A (The target UE side performs monitoring metric calculation) is supported. - Option 2: LMF makes the functionality/model management decision (e.g., functionality/model deactivation, fallback) as a result of model performance monitoring, and notifies the target UE. - Option 2 is at least applicable if Option B (The LMF performs monitoring metric calculation) is supported. Note: From RAN1 perspective, related to model inference and model monitoring of AI/ML based positioning, LMF makes the decision on whether to activate or deactivate positioning methods, including methods using AI/ML positioning. Proposal 6.3.4-1 For label-based model performance monitoring of AI/ML positioning Case 3a, study the following options: - Option 1: the gNB makes the functionality/model management decision/recommendation (e.g., activate or deactivate a model, update a model, fallback to legacy positioning methods) as a result of model performance monitoring. - The gNB notifies the LMF of its functionality/model management decision/recommendation. - Option 1 is at least applicable if Option A (NG-RAN node performs monitoring metric calculation for its own model) is supported. - Send an LS to RAN3 asking whether existing procedures are sufficient to enable this notification. - Option 2: LMF makes the functionality/model management decision (e.g., functionality/model deactivation, fallback) as a result of model performance monitoring, and notifies the gNB. - Option 2 is at least applicable if Option B (LMF performs monitoring metric calculation for the model located at the NG-RAN node) is supported.

During RAN1#119, RAN1 discussed the following proposals on model monitoring.

Proposal 6.1.2-2 For AI/ML based positioning, regarding model performance monitoring, the same entity performs the monitoring metric calculation and model-level monitoring decision making. Proposal 6.1.2-3 For AI/ML based positioning all cases, LMF is responsible for functionality-level performance monitoring and management decision making (e.g., functionality selection, activation, deactivation, fallback to a legacy method).

Performance monitoring can be roughly split between two levels:

- Model-level performance monitoring and management,

- Functionality-level performance monitoring and management.

For the model-level performance monitoring and management, this includes model performance monitoring metric calculation, the decision making on whether the model performs satisfactorily, and actions to take if the model no longer performs well (e.g., deactivate the current model, switch to an alternative model, trigger model update). For this set of procedure, only the model inference entity has the necessary information to carry out, for example, UE for Case 1, gNB for Case 3a. External entities (e.g., LMF) has no knowledge about the details on the model design to make judgements. There are many detailed information on the model that can affect the performance monitoring and management decisions, including:

  - the type/size/granularity of channel measurements used as model input,

  - the model structure (e.g., distributed or centralized model),

  - how the model was trained (e.g., whether the model can accommodate a varying number of active TRPs),

  - the generalization capability of the trained model (e.g., whether/what mixed dataset was used in training),

  - pre-processing and feature engineering applied at model input,

  - postprocessing applied at model output,

  - whether an alternative model is available, and if so, the details of the alternative model,

  - whether a performance degradation is intentional and temporary due to run-time constraints of the model host, e.g., power consumption, memory, processing pipeline, coexistence with other functionalities, etc.

With the above considerations, it is clear that the model-level performance monitoring and management have to be handled by the model inference entity, i.e., UE for Case 1, gNB for Case 3a. In other words, self-monitoring should be supported as a baseline, with the detailed procedure up to the implementation.

For direct positioning, the model inference entity should act as the core entity that performs model monitoring. This includes calculation of model monitoring metrics and making (preliminary) model monitoring decision. To support the model monitoring procedure, assistance information may be provided between nodes, for example, LMF may send LOS/NLOS assistance information to UE for Case 1. This is much more efficient than UE sends raw information about the model input and/or model output to LMF, and LMF calculates the model monitoring metrics for the UE.

For Case 2b and 3b, LMF is the model inference entity. Thus there is no need to define signaling for reporting model performance monitoring decisions. There can be, however, additional signaling to support the LMF decision making in model monitoring. One need is assistance information from UE/gNB to LMF to provide LoS link information (see section 11.1.1). Also LMF could request support from UE/gNB to validate the AI/ML model input.

For assisted positioning, the model inference entity should also act as the core entity that performs model monitoring for itself. To support the calculation of model monitoring metrics, the LMF can help by providing the estimated ground truth to the model inference entity (i.e., UE for Case 2a, gNB for Case 3a), where the estimated ground truth is generated by the LMF using the associated conventional positioning methods.

For functionality-level performance monitoring and management, the LMF is the responsible node in the positioning framework architecture. For Rel-19, LMF should continue to be responsible. That is, with the introduction of AI/ML based positioning, LMF continues to maintain its role as described in TS 38.305:

“For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, gNB positioning capabilities and ng-eNB positioning capabilities. The LMF then invokes these positioning methods in the UE, serving gNB and/or serving ng-eNB. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The LMF may combine all the received results and determine a single location estimate for the target UE (hybrid positioning). Additional information like accuracy of the location estimate and velocity may also be determined.”

In general, LMF is responsible for selecting/activating the desired positioning methods, taking into account the positioning accuracy requirement. LMF manage the positioning functionality, but it does not have detailed information to manage individual models residing at gNB or UE.

Thus, for functionality-level decision making, the LMF is responsible.  RAN2 has agreed to continue with the legacy principle based on LMF-driven functionality management for the positioning method, with UE providing feedback  on whether the functionality is applicable to the UE based on capability reporting or error cause reporting:

Agreements (RAN2#125bis): 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19 Agreements (RAN2#128, Nov 2024) 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

Thus, for both label-based or label-free monitoring, the LMF should still be able to request the UE/gNB to send model-level monitoring updates.

The model inference entity is responsible for model-level performance monitoring and management, including calculation of model monitoring metrics. The detailed algorithm and procedure are up to implementation.

For AI/ML based positioning, LMF is responsible for functionality-level performance monitoring and management.

For Case 1, beyond monitoring the deployed model by UE, some additional validation or supervision may be necessary to ensure a reliable system-level performance. Specifically, LMF may need to check from time to time whether the AI/ML model in UE works well or not. For this purpose, the UE can report model quality information to LMF periodically or when triggered by LMF. The model quality information may include: model monitoring decisions by UE, quality of the model output (e.g., uncertainty and confidence of model output). This allows the LMF to supervise the UE's AI/ML positioning functionality at a high level.

Similarly, for Case 3a, the LMF need to supervise gNB’s AI/ML positioning functionality at a high level. LMF can check the quality of model output reported by gNB. In addition, the gNB can report its model monitoring decisions to LMF periodically or when triggered by LMF. Specifically, the gNB can feedback model monitoring decision to the LMF using e.g. measurement error with cause value. The signaling details are up to RAN3.

in terms of functionality management, the LMF can decide to deactivate the AI/ML based positioning methods, e.g., if the LMF decides that the model is no longer appropriate for the deployment. For example, for Case 1/2a/3a, it can be understood the model inference entity (UE, gNB) performs model monitoring and makes preliminary model monitoring decision, while LMF can make the final decision such as functionality deactivation, fallback to a legacy method. Fallback can be done by implementation in the LMF, no additional specification is needed.

For AI/ML based positioning, LMF can decide on a fallback method based on performance monitoring reports from the UE/gNB

  - Note: there is no specification impact needed to enable a fallback mechanism.

Self-monitoring: Label-free or label-based

During RAN1#119 label-free monitoring was captured in the FL summary with the following proposal:

Proposal 6.1.2-1 For AI/ML positioning Cases, label-free monitoring methods are supported, where the model inference entity performs self-monitoring without external information on the ground truth label. - For Case 1 and Case 3a, the UE (Case 1) and gNB (Case 3a) sends the monitoring decision to LMF, at least when the monitoring decision indicates that the model becomes inappropriate for inference. - FFS: other specification impact (e.g., assistance data).

For AI/ML based positioning, model performance monitoring was studied in Rel-18 study item phase. Both label-based methods and label-free methods were explored. If true ground truth label can be found, it is preferred to use label-based method and monitor the accuracy of model output directly. However, for real life deployment, the problem is, ground truth label or its estimation is rarely available during model deployment.

For label-free model performance monitoring, as discussed in Rel-18 SI, the monitoring metrics can be various formats depending on the method:

- statistics of measurement(s) compared to the statistics associated with the training data,

- statistics associated with the model output,

- relative displacement,

- inference output inconsistency (e.g., approach 3 in section 11.1.3),

- …

There is no effort to acquire ground truth label or its approximation or substitution. The calculation of monitoring metric is up to implementation. It is not feasible to align the monitoring metric among the different monitoring methods, or align on a threshold for decision making at the end of model monitoring. Hence the entire model performance monitoring procedure is self-contained within the model inference entity, i.e., no need to receive assistance data on ground truth label; no need to send monitoring metric to another entity; no need to receive thresholds or criteria for decision making. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

Label-free model performance monitoring is up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

For label-based model performance monitoring, as discussed in Rel-18 SI, the monitoring metric is:

- statistics of the difference between model output and provided ground truth label (or its approximation/substitution).

Thus there is a need to provide ground truth label (or its approximation/substitution).

- If certain information on the ground truth label can be generated by the model inference entity (e.g., a variant of approach 1 of section 11.1.1), then such label-based model performance monitoring is also self-contained and up to implementation of model inference entity.

- On the other hand, if the information on the ground truth label has to be provided by another entity, then signaling needs to be specified so that model output and ground truth label information are brought together to the model monitoring entity. If the model monitoring entity is different than the model inference entity, then signaling is also needed to send the monitoring decision.

  - Still, it is infeasible to align the exact monitoring metric calculation, or align on a threshold for decision making at the end of model monitoring. This imposes unacceptable difficulty to specify them over a standardized interface.

Considering the above, similar to label-free monitoring, for label-based monitoring, a same entity should perform both monitoring metric calculation and monitoring decision making regarding how to handle the model. That is, self-monitoring is the baseline for all cases.  Note that the network (i.e., LMF) is still in charge of the positioning procedure at large (i.e., LMF for functionality-level monitoring), and may decide to interrupt the positioning procedure based on received information.

If certain information on the ground truth label can be generated by the model inference entity, then such label-based model performance monitoring is also self-contained and up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

For AI/ML positioning Cases, LMF based functionality management using label-free monitoring methods are supported, where the model inference entity performs self-monitoring without external information on the ground truth label.

For AI/ML positioning Cases, self-monitoring is the baseline,  where the model inference entity performs self-monitoring with or without external information on the ground truth label (i.e., label-free or label-based).

  - The detailed self-monitoring method and decision-making are up to the implementation of the model inference entity (gNB or UE or LMF).

  - The LMF can request / configure the reporting of monitoring outcome by the model inference entity when it’s gNB or UE.

  - For Case 1 and Case 3a, the UE (Case 1) and gNB (Case 3a) sends the monitoring decision to LMF, at least when the monitoring decision indicates that the model becomes inappropriate for inference.

  - For model inference at UE (Case 1, 2a) or gNB (Case 3a), the LMF can provide assistance information to support the calculation of model monitoring metrics. FFS: what assistance data may be provided.

Model performance monitoring metric calculation for Case 1

Several label-based options for case 1 were proposed for further study

RAN1 #116-bis Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side. - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

In RAN1#118, it was concluded that Option A-4 does not need to be further discussed. RAN1#119 discussed further without agreements, but the following was captured as proposals in the FL summary:

Modified Proposal 6.2.5-3 For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2, A-3 are supported in Rel-19. - From RAN1 perspective, legacy framework, measurements and signaling can be reused for Option A-1, A-2,A-3. - Note: The definition of the monitoring metric is up to UE implementation. Note: No UE capability discussion about supporting A-1, A-2, A-3.

In our view these label-based performance monitoring proposals add complexity to the NR specifications and increase the implementation costs. Some of these proposals even elevates the costs of operating the positioning systems. These label-based monitoring proposals should be compared to label-free monitoring approaches in terms of complexity as well as effectiveness. That is, quantitative evaluation results should be provided for these label-based monitoring proposals, which can be compared to those using label-free monitoring.

Moreover, it is possible to perform label-based monitoring where certain information on the ground truth label can be obtained by the target UE itself. For example, as a variant of monitoring approach 1 of section 11.1.1, the target UE can use a mechanism outside of the Case 1 model to detect opportunistic LoS link(s) and estimate the corresponding distance between the target UE and the TRP, which can then be used as a ground truth label approximation/substitution to monitor Case 1 model.

The more complicated label-based monitoring solutions should be introduced to NR only if the solutions provide added values over those achievable with label-free monitoring solutions.

If the more complicated label-based monitoring is to be supported, then Option A-1 and/or A-2 is sufficient.

- In Option A-1, the LMF derives the ground truth label. Without knowing the UE model implementation, the only ground truth label the LMF can generate is an estimated UE location label. This can be already done using the existing specification, by setting up legacy positioning procedure in parallel to the ML positioning procedure where the UE request the LMF for its location.

- Similarly, Option A-2 can be achieved by setting up legacy UE based positioning in parallel to AI/ML positioning procedure.

Option A-3, B-1, B-2 should not be supported.

- Option A-3 requires the presence of PRU to support model monitoring. This incurs high deployment cost and signaling overhead unnecessarily.  Additionally, periodic provision of PRU measurement as AD to UE based positioning is specified only for CPP in 38.305.

- Option B-1 and B-2 violate the principle of self-monitoring discussed in section 6.2. As discussed in section 6.1, LMF is responsible for functionality-level performance monitoring and management, but it does not have the requisite information to perform model-level management including monitoring metric calculation.

For model performance monitoring of AI/ML positioning Case 1, support Option A-1 and/or A-2 for model performance monitoring metric calculation, if estimated label is considered necessary to be provided by an external entity (which is LMF for Case 1).

For model performance monitoring of AI/ML positioning Case 1, do not support Option A-3, B-1 or B-2.

For AI/ML positioning Cases, if label-based monitoring is supported:

  - The model inference entity performs self-monitoring.

  - The LMF can request / configure the reporting of monitoring outcome by the gNB/UE.

  - For Case 1 and Case 3a, the UE (Case 1) and gNB (Case 3a) sends the monitoring decision to LMF, at least when the monitoring decision indicates that the model becomes inappropriate for inference.

Model inference

Signaling to support model inference

For the model inference stage of LCM, for an AI/ML model to work properly, the model inference context needs to be consistent with model training context. The model training context need to include all the settings that noticeably affect the model performance and cannot be ignored. A model cannot be activated for inference without checking the model inference context requirement. Specifically,

- If the model inference context requirement is satisfied, then the model inference can proceed.

  - This is the ideal case where the trained model is fully compatible with the deployment environment.

- If the model inference context requirement is not satisfied, then configuration can be updated to satisfy the model inference context requirement. After that, the model inference can proceed.

  - This may happen if the model is trained for the deployment area, but some of the parameters (e.g., PRS) are not configured the same as in the model training.

- If the model inference context requirement is not satisfied, and the configuration cannot be updated to satisfy the model inference context requirement, then model inference may not proceed. A notification message (e.g., an error message) can be sent on the failure of model inference due to mismatched context between model training and model inference.

  - This may happen if the deployment area is different from the validity area of the trained model.

Under the broad outline above, there can be further nuanced procedure, if the context requirement is not satisfied for a subset of the TRPs (TRPinconsistent), but satisfied for the remaining TRPs (TRPconsistent). For example, for TRPinconsistent, the PRS beam pattern is changed between training data collection phase and model inference phase. In this case, if the model has been trained to handle a varying number of TRPs and/or a varying set of TRPs, then the model inference can still proceed by including TRPconsistent in model inference, while excluding TRPinconsistent.

The model inference context requirement includes at least the following of an AI/ML model, which are directly related to the model input of the various positioning cases:

  - validity area of the model for inference, e.g., a list of TRPs that transmits PRS (for downlink based cases) or receives SRS (for uplink based bases),

  - configuration of reference signal transmission and measurement,

    - For models using DL PRS (Case 1, 2a, 2b), this refers to DL PRS configuration for transmission (Case 1, 2a, 2b), and measurement report on PRS (Case 2b).

    - For models using UL SRS (Case 3a, 3b), this refers to UL SRS configuration for transmission (Case 3a, 3b), and measurement report on SRS (Case 3b).

More context requirements need to be checked for compatibility, e.g., range of SNR/SINR. See the discussion on metadata of training data in section 5.2.

Consistency aspects in general

In general, the observed fingerprint of a location is the composite picture including transmitter effect, radio channel, and receiver effect. To correctly determine a UE location based on fingerprinting, the features of the composite picture need to stay (approximately) the same for different UEs (e.g., PRU and normal UE) at different times (e.g., training data collection, model inference). Thus it is necessary to ensure consistency in any transmitter behavior (e.g., PRS configuration) and receiver behavior (e.g., measurement type and granularity, at least for Case 2b/3b) that affect the observed fingerprint.

Similar to the consistency issues discussed for the training data collection phase (see section 5.2), for UE-side model, at the following need to be checked for consistency between training and inference.

    - Validity area (section 7.2.1)

    - Assistance data. This includes:

      - Explicit configuration for RS reception (section 7.2.2)

      - Associated ID for network-side additional conditions (section 7.2.3)

    - Signal measurement configuration (section 7.2.4)

Consistency of validity area

Regarding deployment environment identifiers, the model inference validity area Ainference needs to be recorded for an AI/ML model when model training is performed. When the trained model is checked for potential activation, the model inference validity area Ainference can be checked against the deployment environment. It can be defined as:

| Model inference validity area | The network area for which the model inference is valid. |
| --- | --- |

The model inference validity area Ainference may or may not be different from the training dataset validity area Adataset mentioned in section 5.2.2.

Training dataset validity area Adataset: this describes the area that training dataset is collected. Adataset is from the perspective of training data collection entity, as explained in section 5.1.2. For example, in Figure 4, the training data includes measurements associated with 18 TRPs, TRP {0,1,…,17}. Hence the training dataset validity area Adataset = TRP {0,1,…,17} covers the entire factory floor.

Model inference validity area Ainference: this describes the coverage area that the trained model is used to perform inference for. Ainference is from the perspective of model inference entity. Ainference can be the same as Adataset, or a subset of Adataset. For example, in Figure 4, the model inference entity may use measurements associated with 9 TRPs on the left hand side for inference, Ainference= TRP {0,1,…,8}, which is a sub-area of Adataset.

Figure 4. An example where the model inference validity area is a sub-area of training dataset validity area.

It should be clarified that the model inference validity area is a property of the trained model. It shall not be confused with the legacy IE assistanceDataValidityArea-r17, which is for the validity area of the assistance data for the associated legacy method, see below for DL-TDOA and DL-AoD.

assistanceDataValidityArea This field specifies the network area for which this NR-DL-TDOA-ProvideAssistanceData is valid.

assistanceDataValidityArea This field specifies the network area for which this NR-DL-AoD-ProvideAssistanceData is valid.

Another difference is, for model inference validity area, a cell or a TRP ID may belong to two different areas, i.e., two different areas can overlap. In contrast, two different assistanceDataValidityArea cannot overlap, i.e., a cell can appear in at most one assistanceDataValidityArea (see AreaID-CellList description: “Each cell is included in only one area.”). See explanation in section 5.2.2 as well.

In terms of signaling, the validity area does not require any new IE at model inference stage. The UE simply checks the observed TRP list against the model inference validity area Ainference, which has been stored as a part of metadata for the trained model.

The model training entity records the model inference validity area as a part of metadata for the trained model.

When preparing for model inference, the model inference validity area is checked for compatibility with the deployment area.

Consistency of RS configuration between model inference and model training

For configuration of reference signal transmission and measurement, DL PRS and UL SRS need to be addressed for the relevant positioning cases. When the AI/ML model takes DL PRS measurement as input, for configuration of PRS transmission and measurement, it is necessary to ensure that the same configuration is used in training data collection stage and model inference stage. Also, the same configuration is used for all PRUs and UEs, for example, the IDs (e.g., TRP ID, dl-PRS-ID, DL PRS Resource Set ID, a DL PRS Resources ID) are assigned the same to all PRUs and UEs, and not varying in a UE-specific manner.

Similarly, when the AI/ML model takes UL SRS measurement as input, for configuration of SRS transmission and measurement, it is necessary to ensure that the same configuration is used in training data collection stage and model inference stage.

When preparing for model inference, configurations of reference signal transmission and measurement are checked for compatibility between the trained model and the deployment environment.

In terms of specification impact, for Case 2a/3a the model readiness for inference may be signaled from the model inference node (e.g., UE/gNB) to other nodes requesting the measurements, e.g. the LMF, in order to signal that AI/ML positioning is available.

For Case 2a/3a, support UE/gNB to send an indication to the LMF when the AI/ML model is ready for reporting measurements based on model inference. It is up to RAN2/3 to decide which procedure to use to convey the information.

Ideally, signals configured for inference are configured exactly the same for the training phase. For example, PRS configuration of bandwidth, comb offset, etc., should remain unchanged between inference and training. To make sure of this, the training data could be generated for all possible signal configurations, but this would lead to a massive training data set.  Thus some misalignment may be unavoidable. For example, the SRS resource used for training the gNB based model used in Case 3a may be already assigned to another UE at the time of inference, so that a resource with a different comb offset and or cyclic shift may be chosen to be configured.  It is however important that the measured channel response at inference is consistent with the one measured during training. This means that at least bandwidth, transmitted power and spatial properties of the reference signals stay the same between inference and training.

In summary, before model inference can start, the UE/gNB should ensure the following stay unchanged between training and inference:

- PRS:

  - Positioning frequency layer configuration (bandwidth, comb size, SCS, starting PRB, CP)

  - Muting patterns

  - DL PRS spatial properties

  - Power over the measured bandwidth

- SRS:

  - Bandwidth configuration (starting PRB, bandwidth, comb size, SCS, CP)

  - UL SRS for positioning spatial properties

  - SRS transmission power

  - UL path loss reference

Consistency of network-side additional conditions between model inference and model training

For AI/ML based positioning with UE-side model, to support verification of NW additional conditions, associated IDs can be defined to identify a given set of NW additional conditions. This has the benefit that the IE details do not need to be explicitly signaled, if they are not needed for performing model inference, and only needed for consistency verification. This avoids exposing unnecessary network condition details and reduces signaling overhead.

The benefit of associated ID is that the details for the information covered by associated ID do not need to be explicitly signaled, if they are not needed for performing model inference, and only needed for consistency verification.

Adopting associated ID for the positioning use case is similar to the agreement made in RAN1#118 for beam management (AI 9.1.1):

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

In RAN1#118, the following working assumption was confirmed under agenda item 9.1.3.3.

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

One difference between the positioning use case and beam management use case is, for BM the model covers the beams in a given cell only, while multiple cells/TRPs are involved in the model for positioning. Also, the number of TRPs that are needed for generating model input can vary, if a model for positioning is trained to support a varying number of TRPs.

Thus for the positioning use case, the coverage of associated ID should be discussed separately. In our view, the consistency between training and inference should be checked for each TRP separately, including the NW-side conditions explicitly signalled (e.g., PRS bandwidth) and the NW-side additional conditions which are covered by associated IDs.

Regarding the associated ID, it is to be defined per TRP, i.e., for a given TRP j, the UE assumes that the NW-side additional conditions with the same associated ID are consistent within the TRP j.

- If for a given TRP j, the same associated ID is used between training and inference, then TRP j may be included in model inference by the UE.

- If for a given TRP i, different associated IDs are used between training and inference, then TRP i should not be included in model inference by the UE.

For example, for info #7 (Geographical coordinates of the TRPs), the associated ID of each TRP indicates whether the geographical location of the given TRP has changed or not.

RAN1 should further discuss the range of values for associated ID (hence the number of bits to represent the associated ID).

For UE-side model (Case 1/2a), to ensure the consistency of NW-side additional condition across training and inference, support associated ID for indicating NW conditions/configurations.

For AI/ML based positioning, the consistency between training and inference is checked for each TRP separately.

For a given TRP j, the UE assumes that the NW-side additional conditions with the same associated ID are consistent within the TRP j.

Consistency of measurements reported between model inference and model training

For the consistency between the RSs used during training and inference, similarly it could be left to network deployment choice at least for cases 1/2a/3a. Although there could be several types of UE deployed in the network and each could be using e.g. a different bandwidth during training, parameters can be controlled by network implementation to remain static during both training and inference. The UE can also ensure the measurement bandwidth is consistent with the bandwidth used for training. For case 3b and 2b, the LMF gather the measurements and the gNB/UE may not have been involved with the model training.  It is thus important that the LMF signals what measurement bandwidth to use to provide model input.

For Cases with LMF-side model (Case 2b/3b), the measurements reported for model input must be consistent with the ones used with training data.

For measurements reported to the LMF in case 2b/3b, support the LMF to indicate the PRS/SRS measurement bandwidth.

Consistency of network side conditions for Case 1

The following agreement was reached on the assistance data required to maintain consistency in case 1:

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

Hence the remaining issue is whether the TRP location is required to case 1, and if so how to signal it.  Additionally,  there are proposal to also support AOD assistance data.  As discussed in section 1 of this contribution, only time-based methods have been discussed in the SI phase, and we thus assume only assistance data from time-based methods as baseline for Case 1.  Furthermore, one should consider what is strictly necessary for case 1, which is essentially a fingerprinting method. This most notably changes the need the UE has in terms of calculation assistance data. Therefore, info #16-18 are not to be provided explicitly from LMF to UE, which are those that are specified for UE-based DL-AoD but not UE-based DL-TDOA. For example, for spatial information, info #6 is adequate for DL-TDOA, and info #16 provides further details that DL-AoD needs, where info #6 is “Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB”.

Table 8. Information that are specified for UE-based DL-AoD but not UE-based DL-TDOA

|  | Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TdoA [1] or UE-based DL-AoD [2], as applicable | UE-based DL-TDOA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) | N/A | Explicit |
| 17 | Expected Angle Assistance information | N/A | Explicit |
| 18 | PRS priority list | N/A | Explicit |

For Case 1, the SI only considered time-based fingerprinting approaches.

Do not support provision of assistance data (info #16-18) dedicated to UE-based DL-AoD, but not needed for DL-TDOA.

Regarding the signalling of TRP location, we think that one should consider firstly whether the information is essential to AIML fingerprinting methods, and also consideration of sensitive information for operators.  Pre release-19 methods for DL-TDOA compute the time of arrival based on tri-alterations for which the coordinates of the TRPs transmitting the reference signal are needed to resolve the location equation.  For fingerprinting, the location coordinates of the TRP are not needed, since this information is implicitly imbedded in the label data of the training dataset, and never used explicitly in any calculation. Thus it is enough to know whether the TRP position has changed during the inference phase and during the model training phase.  Similar approach could also be taken for spatialinformation, or fine timing relative to the serving TRP.

Hence, strictly speaking, absolute location knowledge of the TRP location is not necessary (contrary to Alternative 4). Moreover, leaving Alternative 4 as the only option excludes operators for which such information is sensitive.   At a minimum, RAN1 should leave the choice to the operator to use a consistency mechanism like associated ID, or the explicit coordinate of the TRP.  The UE may request either of the two types of TRP location information, but it should be left to the operator to decide which one to eventually provide.

For Case 1, operators should have the possibility to support Case 1 and without disclosing TRP location.

Based on the observations above, Alternative 2 seems to be a reasonable compromise. Alternatively, to preserve operators sensitive information and enable case 1 the other alternative is to not signal the TRP location at all, i.e. Alternative 3.

For case 1, regarding info#7 in the assistance data from DL-TDOA, support Alternative 2

  - Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

    - Note: if explicit signaling is not supported by the network, legacy behavior can be used to exclude the TRPs without provided location from provided AD.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

Conclusion

In the previous sections we made the following observations:

Observation 1	For Case 1/2a (i.e., UE-side model), channel measurement generation process is collocated in the same node with the AI/ML models. The exact details of the model inputs are implementation issues at least for model inference.

Observation 2	For Case 3b (1st priority) and Case 2b (2nd priority), due to the LMF sided model, there is a need to clarify the target model input formats to be supported for the AI/ML positioning models.

Observation 3	From the candidate values agreed in RAN1#119 for Nt (Nt = {32, 64, 128} ), Nt = 128 does not provide a significant gain.

Observation 4	For Case 3b measurement reporting, gNB measurement report capability may vary over time.

Observation 5	For Case 3b, automatic fallback from one measurement method to the other cannot be guaranteed.

Observation 6	When compared to the total-power PDP input type, the 2-port PDP input type (1) doubles the signal sizes; (2) requires higher computational complexity; and (3) achieves marginal performance improvements.

Observation 7	When compared to the total-power PDP input type, the 1-port PDP input type (1) discards signal power and radio channel information that is readily available, and (2) achieves lower positioning accuracy.

Observation 8	Rel-18 CPP measurements (DL RSCPD, DL RSCP, UL RSCP) assume LoS environment, which is contradictory with the target use case of Rel-19 AI/ML based positioning.

Observation 9	Single phase value for first path/sample (e.g., DL RSCPD, DL RSCP, UL RSCP) brings no observable performance benefits to PDP, but incurs substantially high deployment cost and signalling overhead.

Observation 10	The initial phase values of measured CIR samples may be random and contains no useful spatial dependent information for an AI/ML model to learn the association between measured CIR samples and the target positions. Without addressing such spurious information during training and inference, fingerprinting ML models using the CIR inputs can produce inaccurate position estimates.

Observation 11	CIR samples containing relative phases can be used as inputs to AI/ML models to circumvent the impact of random initial phase values of measured CIR samples.

Observation 12	For the RSCPD measurement PRU assisted phase error compensation requires PRUs to be used during training and inference, further increasing the deployment cost and signaling overhead.

Observation 13	For inter-path phase measurements, the model input at training and inference must know the UE TX or RX antenna phase contribution, either by reporting the antenna phase contribution to the model, removing it, or assuming the same antenna pattern is used during inference and training.

Observation 14	It is unclear how a mobile UE would know the direction of departure / arrival and associated phase offset for each path toward a given TRP.

Observation 15	For small or moderate signaling sizes, PDP and DP samples can achieve better positioning accuracy than CIR samples at the same or smaller signaling size. Multi-port CIR samples can achieve higher positioning accuracy only with very large signaling requirements.

Observation 16	For small or moderate signaling sizes, PDP and DP samples can achieve better positioning accuracy than CIR samples at the same or smaller signaling size as well as with substantially lower AI/ML model complexity.

Observation 17	For most given 90%tile 2D UE positioning error requirements, the DP samples requires the smallest signaling sizes.

Observation 18	Multi-port complex-valued CIR samples for both Case 3b (1st priority) and Case 2b (2nd priority) require very large signaling sizes, which can cause significantly negative impacts on the radio and core networks.

Observation 19	With small to moderate number of samples available to an ML model, the total-power PDP input type is a very helpful induction bias to impose on the ML model based on human domain knowledge that the sample powers contain more important information about the UE positions. It’s only with very large number of samples that the model can start to tease out how to use the additional information in the sample phases on its own.

Observation 20	Dimension reduction techniques can be used to reduce the signal sizes of multi-port CIR samples. But the achievable positioning accuracy is also compromised. The overall accuracy vs overhead tradeoff situation of CIR samples is not improved by the two considered dimension reduction techniques.

Observation 21	For Case 3a, the location of the AI/ML model (e.g., gNB-DU vs gNB-CU) impacts the need to specify new measurements for model input.

Observation 22	The LOS/NLOS indicator in legacy positioning report is a measure of the reliability of the measurement report.

Observation 23	AI/ML assisted positioning can provide measurement report of the NLOS channel with high confidence.

Observation 24	The interpretation of a LOS indicator as reflecting the physical nature of the channel should be the same in Release 19 and legacy.

Observation 25	If the LMF is not involved in the model monitoring of the model generating the LOS indicator, the need for identifying a LOS indicator as generated by a model output is not clear.

Observation 26	From the RAN1 perspective, current specification support for Part A data collection in Case 1 is sufficient.

Observation 27	From the RAN1 perspective, Part A data collection in Case 2b and 3b is identical to measurement reporting for inference.

Observation 28	From the RAN1 perspective, Part A data collection in Case 3a is either up to the network implementation or identical to measurement reporting for inference.

Observation 29	Time stamp for Part B may span a period of time for which the location of the UE is consistent.

Observation 30	Part A-B pairing is an issue across Case 1, 3a and 3b.

Observation 31	Part A and Part B must be paired for the same UE, even though Part A and Part B may be reported by different entities.

Observation 32	Collecting sufficient data in terms of training dataset size, UE distribution, diversity of UE sources, etc, is up to implementation.

Observation 33	Label-free model performance monitoring is up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

Observation 34	If certain information on the ground truth label can be generated by the model inference entity, then such label-based model performance monitoring is also self-contained and up to implementation of model inference entity. The only potential specification impact is to notify other entities of the model monitoring decision, if necessary.

Observation 35	The benefit of associated ID is that the details for the information covered by associated ID do not need to be explicitly signaled, if they are not needed for performing model inference, and only needed for consistency verification.

Observation 36	For Cases with LMF-side model (Case 2b/3b), the measurements reported for model input must be consistent with the ones used with training data.

Observation 37	For Case 1, the SI only considered time-based fingerprinting approaches.

Observation 38	For Case 1, operators should have the possibility to support Case 1 and without disclosing TRP location.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 assumes Case 1 is supported using UE based DL-TDOA procedures as a foundation. It is up to RAN2 to decide whether a set of new procedure is to be defined for Case 1, or extension to DL-TDOA suffices. RAN1 confirms the understanding with RAN2.

Proposal 2	RAN1 assumes Case 3a is supported using UL-TDOA or multi RTT procedure as a foundation. It is up to RAN3 to decide whether a set of new procedure is to be defined for Case 3a, or extension to existing procedure suffices. RAN1 confirms the understanding with RAN2 and RAN3.

Proposal 3	RAN1 assumes that for Case 3b with sample-based reporting enhancements, measurement report with sample-based reporting can be differentiated by the involved nodes (LMF, gNB) from the legacy UL RTOA measurement requests / reports. RAN1 confirm the understanding with RAN2 and RAN3.

Proposal 4	From the candidate values agreed in RAN1#119 for Nt (Nt = {32, 64, 128} ), support Nt = {32, 64}.

Proposal 5	From the candidate values agreed in RAN1#119 for Nt' (Nt' <=24), Nt' values {9, 16, 24} are supported.

Proposal 6	For {Nt, Nt'}, support these combinations: {32, 24}, {32, 9}; {64, 24}, {64, 16}, {64, 9}.

Proposal 7	For Case 3b (and 3a if needed), a small number of k values, e.g., k={4}, k={3,4}, or k={2,3,4,5}, is supported for timing reporting granularity factor in Rel-19.

Proposal 8	For Case 3b (and 3a if needed), when enhanced measurement are reported (i.e., sample-based reports) from the gNB to the LMF:

	When the Nt, Nt', k values used to produce the report are different from the ones provided by the LMF in the measurement request, gNB provides in the measurement report to the LMF the Nt value used for the measurement.

	Note: k value is already supported in the measurement report, and Nt’ can be inferred by the report size.

Proposal 9	For Rel-19 AI/ML based positioning Case 3b, LMF includes an indicator in the measurement request to gNB, where the indicator is on the LMF preferred measurement report: either (a) sample-based measurement, or (b) legacy path-based measurement.

Proposal 10	For Rel-19 AI/ML based positioning, legacy path-based measurement (if needed) is used without enhancement.

Proposal 11	For Case 3b, do not specify signalling enhancements for fallback between sample-based and path-based measurement.

	Note: fallback can be supported by existing procedure using cause values.

Proposal 12	Regarding PDP for model input, update the DL PRS-RSRPP and UL SRS-RSPP definitions where the path or sample powers are summed over all receive antenna ports, i.e., using total-power PDP.

Proposal 13	Do not support phase information for determining model input, including CIR and single phase value for first path/sample.

Proposal 14	Before CIR can be adopted as model input, RAN1 need to investigate whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits.

Proposal 15	To decide whether to support phase information to enable CIR as model input, RAN1 should weigh the small positioning accuracy improvement against the standardization effort, the signaling overhead, and the difficulty to align phase information between training and inference.

Proposal 16	If phase information is supported to enable CIR as model input, the phase values are reported in the format of relative phase.

Proposal 17	RAN1 to down-prioritize the signaling approach(es) and/or measurement definitions to support CIR model input types for Case 3b (1st priority) and Case 2b (2nd priority).

Proposal 18	Send an LS to RAN3 to request feedback on whether the input to the AI/ML model for case 3a need to be specified.

Proposal 19	For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, the timing information is for virtual LOS link if carried in a new Rel-19 measurement report, and the timing information is for physical LOS link if carried in a legacy measurement report (Rel-18 or prior).

Proposal 20	For Rel-19 measurement report, the LOS/NLOS indicator (for physical LOS link) is decoupled from the timing information reporting (for virtual LOS link).

Proposal 21	Support reporting (optionally) a LOS/NLOS indicator when timing information is reported for Rel-19 AI/ML positioning Case 3a.

Proposal 22	For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, from RAN1 perspective, the LOS/NLOS indicator provides information on the likelihood of a physical Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LOS and a value of 0 corresponding to NLOS; the LOS/NLOS indicator reuse the same format as the existing IE "LOS/NLOS Information" in 38.455.

Proposal 23	(for conclusion) For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, do not support an indication of whether the LOS/NLOS indicator is obtained via a model output or via legacy method.

Proposal 24	From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, one LOS/NLOS indicator is optionally reported per TRP in a measurement report from gNB to LMF, as in the existing specification.

Proposal 25	For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, LMF shall be able to distinguish whether the timing information is obtained for Rel-19 measurement report (virtual LOS link) or for legacy measurement report (physical LOS link).

	For timing information obtained for Rel-19 measurement report, If a timing measurement is reported together with the LOS/NLOS indicator, the indicator status (for physical LOS/NLOS) is decoupled from the reported timing measurement (for virtual LOS)

	Note: when the TRP-UE radio link is LOS, the virtual LOS coincides with the physical LOS.

	Details of how to associate the Rel-19 timing information with virtual LOS is up to RAN2/3, e.g. indication in measurement report, or configuration in measurement request.

Proposal 26	Reuse the existing measurement reporting in LPP and NRPPa to report timing information for the model output of AI/ML assisted positioning.

Proposal 27	For AI/ML assisted positioning at gNB (Case 3a), the model output is uplink relative time of arrival (TUL-RTOA).

Proposal 28	For AI/ML assisted positioning at gNB (Case 3a), postprocessing is applied to generate gNB RxTxTimeDiff for measurement reporting of multi-RTT method.

Proposal 29	For AI/ML assisted positioning at gNB (Case 3a) and UE (Case 2a), the measurement report contains an indicator that AI/ML is used to produce the measurements. The signaling details are up to RAN3/RAN2.

Proposal 30	For Rel-19 AI/ML based positioning, support collecting labelled and optionally un-labelled training data samples.

Proposal 31	For training data collection of AI/ML based positioning, for both Part A and Part B, time stamp is mandatorily provided by the training data generation entity.

Proposal 32	For training data collection of AI/ML based positioning, the time stamp should allow unambiguous mapping between Part A and Part B.

	For channel measurement generated by TRP/gNB, at least when measurement periodicity exceeds the SFN/slot wrap around time (10.24 sec), field “Measurement time” is included in the time stamp.

	For channel measurement and label generated by PRU or non-PRU UE, UTC Time (e.g., for utc-time-r16) is included in the time stamp.

	Exactly how to specify time stamp for Part A and Part B is up to RAN2/RAN3.

Proposal 33	From RAN1 perspective, the existing IEs and their associated granularities can be reused for time stamp of Part A and Part B. It is acceptable to have different time stamp granularity for Part A and Part B. There is no need for RAN1 to discuss how to perform the pairing using the time stamps.

Proposal 34	RAN1 does not need to discuss further signalling to enable Part A data collection.  Measurement reporting is jointly discussed for training data collection and inference.

Proposal 35	For training data collection in all cases, a power quality is provided for the entire PDP measurement in Part A in units of dB. The candidate resolution includes 0.1 dB and 1 dB.

Proposal 36	Confirm the existing WAs on label generation for all cases and send the agreements to RAN2.

Proposal 37	For training data collection Part B, support only ground truth labels in the format of UE location coordinates (x, y, z).

Proposal 38	From RAN1 perspective, the same label (UE location) and label quality (UE location uncertainty) are collected, irrespective of the type of model architecture (e.g., assisted or direct positioning) that may be selected by the model training entity.

Proposal 39	For training data collection, the label quality for Part B reuse the existing IEs for indicating UE location quality. From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The uncertainty and confidence serve as quality indicator of the label.

Proposal 40	A label provided by a LMF, PRU, or non-PRU UE can be attached with one or more time stamp, or a span of time stamps, for which the label is valid.

Proposal 41	Send an LS to RAN2 and RAN3 requesting to investigate how Part A and Part B can be matched to the same UE for Case 1/3a/3b.

Proposal 42	The training dataset is attached with metadata which describes the context information on the set of collected training data samples.

Proposal 43	For training data collection of Part A, the metadata includes: Group 1 for assistance data, Group 2 for signal measurement configuration, and Group 3 for measurement environment condition and quality.

Proposal 44	RAN1 to study the details of Group 3 metadata to attach to training dataset, including:

	Range of SNR/SINR

	Range of Channel estimation error

	Range of NW synchronization error

	Range of UE/gNB RX and TX timing error

Proposal 45	The training dataset validity area is recorded as a part of metadata of the training dataset. The validity area is defined as a list of TRPs where data is collected.

Proposal 46	Training data provided to UE/gNB is organized with the same hierarchy as for legacy assistance data, i.e. per TRP/frequency layers for the UE, and per TRP for the gNB assistance data.

Proposal 47	For Part A training data collection of Case 1/2a, the assistance data include: training dataset validity area, assistance data necessary for PRS reception, and associated ID for additional network conditions. Corresponding assistance data is sent during model inference as well for consistency checking.

Proposal 48	The assistance data required for PRS/SRS reception is the same across all use cases for both direct and AI/ML assisted positioning.

Proposal 49	The model inference entity is responsible for model-level performance monitoring and management, including calculation of model monitoring metrics. The detailed algorithm and procedure are up to implementation.

Proposal 50	For AI/ML based positioning, LMF is responsible for functionality-level performance monitoring and management.

Proposal 51	For AI/ML based positioning, LMF can decide on a fallback method based on performance monitoring reports from the UE/gNB

	Note: there is no specification impact needed to enable a fallback mechanism.

Proposal 52	For AI/ML positioning Cases, LMF based functionality management using label-free monitoring methods are supported, where the model inference entity performs self-monitoring without external information on the ground truth label.

Proposal 53	For AI/ML positioning Cases, self-monitoring is the baseline,  where the model inference entity performs self-monitoring with or without external information on the ground truth label (i.e., label-free or label-based).

	The detailed self-monitoring method and decision-making are up to the implementation of the model inference entity (gNB or UE or LMF).

	The LMF can request / configure the reporting of monitoring outcome by the model inference entity when it’s gNB or UE.

	For Case 1 and Case 3a, the UE (Case 1) and gNB (Case 3a) sends the monitoring decision to LMF, at least when the monitoring decision indicates that the model becomes inappropriate for inference.

	For model inference at UE (Case 1, 2a) or gNB (Case 3a), the LMF can provide assistance information to support the calculation of model monitoring metrics. FFS: what assistance data may be provided.

Proposal 54	For model performance monitoring of AI/ML positioning Case 1, support Option A-1 and/or A-2 for model performance monitoring metric calculation, if estimated label is considered necessary to be provided by an external entity (which is LMF for Case 1).

Proposal 55	For model performance monitoring of AI/ML positioning Case 1, do not support Option A-3, B-1 or B-2.

Proposal 56	For AI/ML positioning Cases, if label-based monitoring is supported:

	The model inference entity performs self-monitoring.

	The LMF can request / configure the reporting of monitoring outcome by the gNB/UE.

	For Case 1 and Case 3a, the UE (Case 1) and gNB (Case 3a) sends the monitoring decision to LMF, at least when the monitoring decision indicates that the model becomes inappropriate for inference.

Proposal 57	The model training entity records the model inference validity area as a part of metadata for the trained model.

Proposal 58	When preparing for model inference, the model inference validity area is checked for compatibility with the deployment area.

Proposal 59	When preparing for model inference, configurations of reference signal transmission and measurement are checked for compatibility between the trained model and the deployment environment.

Proposal 60	For Case 2a/3a, support UE/gNB to send an indication to the LMF when the AI/ML model is ready for reporting measurements based on model inference. It is up to RAN2/3 to decide which procedure to use to convey the information.

Proposal 61	For UE-side model (Case 1/2a), to ensure the consistency of NW-side additional condition across training and inference, support associated ID for indicating NW conditions/configurations.

Proposal 62	For AI/ML based positioning, the consistency between training and inference is checked for each TRP separately.

Proposal 63	For a given TRP j, the UE assumes that the NW-side additional conditions with the same associated ID are consistent within the TRP j.

Proposal 64	For measurements reported to the LMF in case 2b/3b, support the LMF to indicate the PRS/SRS measurement bandwidth.

Proposal 65	Do not support provision of assistance data (info #16-18) dedicated to UE-based DL-AoD, but not needed for DL-TDOA.

Proposal 66	For case 1, regarding info#7 in the assistance data from DL-TDOA, support Alternative 2

	Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

i.	Note: if explicit signaling is not supported by the network, legacy behavior can be used to exclude the TRPs without provided location from provided AD.

	If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

References

3GPP TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN.

3GPP TS 37.355, LTE Positioning Protocol (LPP).

3GPP TS 38.455, NR Positioning Protocol A (NRPPa).

3GPP TR 38.843 V2.0.1 (2023-12), Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).

RP-243244, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," Qualcomm, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

R1-2306454, “Evaluation of AI/ML for Positioning Accuracy Enhancement,” Ericsson.

Appendix A. High accuracy uncertainty and confidence as defined in TS 23.032

6.2a	High Accuracy Uncertainty

The high accuracy uncertainty r, expressed in metres, is mapped to a number K, with the following formula:

r=C1+xK−1

with C = 0.3 and x = 0.02. With 0  K  255, a suitably useful range between 0 and 46.49129 metres is achieved for the high accuracy uncertainty, while still being able to code down to values as small as 6 millimetre. The uncertainty can then be coded on 8 bits, as the binary encoding of K.

Table 6.2a-1: Example values for the high accuracy uncertainty function

| Value of K | Value of uncertainty |
| --- | --- |
| 0 | 0 m |
| 1 | 0.006 m |
| 2 | 0.01212 m |
| - | - |
| 20 | 0.14578 m |
| - | - |
| 40 | 0.36241 m |
| - | - |
| 60 | 0.68430 m |
| - | - |
| 80 | 1.16263 m |
| - | - |
| 100 | 1.87339 m |
| - | - |
| 120 | 2.92954 m |
| - | - |
| 127 | 3.40973 m |
| - | - |
| 255 | 46.49129 m |

6.5	Confidence

The confidence by which the position of a target entity is known to be within the shape description, (expressed as a percentage) is directly mapped from the 7 bit binary number K, except for K=0 which is used to indicate 'no information', and 100 < K ≤128 which should not be used but may be interpreted as "no information" if received.

Appendix B. Model performance monitoring approaches

In the following, we review three highly effective performance monitoring solutions, which are applicable to Case 1 and Case 3a.

Case 1 is UE-based positioning with UE-side model, direct AI/ML positioning, where an AI/ML model taking multiple measurement inputs to produce an estimate of the UE position, as illustrated in Figure 5(a).

Case 3a NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning. An AI/ML model taking multiple measurement inputs to produce multiple model outputs such as the ToAs to multiple TRPs. The ToAs can then further processed to produce an estimate of the UE position using, for example, conventional triangulation optimization. This is illustrated in Figure 5(b).

(a) direct AI/ML positioning				(b) AI/ML assisted positioning

Figure 5. Two possible implementations of Case 1 UE-based positioning with UE-side model.

In Section 11.1.1 (label-based) and 11.1.3 (label-free), we provide two effective performance monitoring methods which can be applied for both the direct positioning (e.g., Case 1) and the assisted positioning (e.g., Case 3a). In Section 11.1.2, we provide an effective label-based performance monitoring for assisted positioning (e.g., Case 3a). The discussion shows that self-contained performance monitoring is possible for both Case 1 and Case 3a, with either (a) label-free monitoring methods or (b) label-based monitoring methods with certain label related information generated by the model inference entity itself.

Specific to Case 1, the method in Section 11.1.1 is an example of target UE perform label-based monitoring without label related info from other entities; the method in Section 11.1.2 is an example of label-free monitoring. From the discussion, we can conclude the following for Case 1:

Self-contained performance monitoring solutions for Case 1 have been shown to achieve high effectiveness in AI/ML model performance assessment. These performance results are to be used as baselines to evaluate the complexity vs added benefits tradeoff for the label-based performance monitoring proposals under consideration in RAN1.

Monitoring approach 1 (Label-based): Opportunistic LoS links

In a cluttered environment, there is often a low probability of line-of-sight for a radio link between a UE and a TRP. For example, for InF-DH environment, Table 9 shows the LoS probabilities of a radio link between TRP and UE when assuming different InF-DH clutter parameter settings. It is observed that the LoS probability of a UE-TRP link ranges from 44.9% in a mildly cluttered environment to only 0.8% in a heavily cluttered environment. For InF-DH with clutter parameter {60%, 6m, 2m}, line-of-sight links are very rare (probability=0.8%) and this is the reason why conventional positioning methods fail.

Table 9. LoS probabilities of a radio link between TRP and UE when assuming different InF-DH clutter parameter settings.

| Environment clutter parameter setting | LoS Probability |
| --- | --- |
| {40%, 2m, 2m} | 0.449 |
| {50%, 2m, 2m} | 0.352 |
| {60%, 2m, 2m} | 0.268 |
| {40%, 6m, 2m} | 0.014 |
| {50%, 6m, 2m} | 0.025 |
| {60%, 6m, 2m} | 0.008 |

On the other hand, Table 9 shows that LoS link can still be encountered from time to time, even for the most cluttered environment of InF-DH with clutter parameter {60%, 6m, 2m}. In particular, considering that there are 18 TRPs that the UE monitors, the probability of observing at least one LoS link is actually great than 10%.

In Case 1, the target UE measures the PRS from the TRPs and determines an estimate, pUE, of its own positioning using a direct positioning AI/ML model. The target UE can also

- Identify the LoS links using either (1) existing LOS/NLOS classification implementation supporting Rel-17 positioning, or (2) an AI/ML model for LOS/NLOS classification.

- Estimate the ToA, tLOS, for the identified LoS link using existing LOS/NLOS classification implementation supporting Rel-17 positioning, or (2) an AI/ML model (which can be the same or different than the model for LOS/NLOS classification).

Rel-17 specs supports the network providing the TRP locations to the UE. Using the UE position estimates and the TRP location of the LoS link, the UE can calculate the distance to the LoS TRP.

The UE can form an AI/ML model performance metric on its own direct positioning AI/ML model as

distpUE, pTRPLOS−c⋅tLOS

Figure 6. An example of UE in a cluttered location where there is only one LoS link to TRP4.

In the following, we provide the evaluation results of such opportunistic LoS link based performance monitoring solution. Figure 7 shows the cumulative distribution function (CDF) of the AI/ML model performance metric described in the above.

- The direct positioning model is trained using the data collected from the indoor factor scenario with 60% clutter density, and clutter height and width of 6 m and 2 m, respectively (i.e., InF-DH with clutter parameter {60%, 6m, 2m}).

- The LOS/NLOS classification model is an with a computational complexity of less than 1M FLOPs. The model has been tuned to avoid false LoS classification and has a LoS miss probability of 27%. This model is used here since it was found to be effective working on a single TRP-UE link, where it generates LOS/NLOS classification, and ToA estimation if the link is classified as LoS.

Two curves are shown. One curve is for "same environment", which means that the inference environment is the same as the training environment for the inference model. The inference model performs normally in "same environment", i.e., no performance degradation. The other curve is for "new environment", which means that the inference environment is different from the training environment for the inference model, though both environments belong to InF-DH with clutter parameter {60%, 6m, 2m}, i.e., different random seeds were used. The inference model performs poorly in "new environment", i.e., performance degradation has occurred, and model performance monitoring method should detect it.

It can be observed that the CDF of the LoS link ToA difference of "same environment" is much smaller (more than an order of magnitude) and easily distinguishable from the CDF of "new environment". Thus model performance monitoring decision can be made based on the easily distinguishable behavior.

Since the model inference environment is the heavily NLoS environment of InF-DH with clutter parameter {60%, 6m, 2m}, the LoS probability is low and inadequate for conventional positioning methods, as expected. However, approximately 10% of positioning requests (containing measurement samples for 18 TRP links) can be opportunistically leveraged for performing model monitoring, i.e., at least one link out of the 18 is LoS. This is adequate for the purpose of model performance monitoring.

Figure 7. Cumulative distribution function of the difference of the ToA reports generated by the inference model and the monitoring model for the links classified as LoS by the monitoring model in two different realizations of InF-DH with clutter parameter {60%, 6m, 2m}.

This evaluation result demonstrates that it is effective to monitor AI/ML model performance using opportunistic LoS links even in a heavily NLoS environment like InF-DH with clutter parameter {60%, 6m, 2m}.

In general, the key is to provide reliable LoS link identification to the model inference entity, regardless of how such information is obtained. For instance, non-AI methods (e.g., legacy methods in existing specification) can also be used to provide the LOS/NLOS classification as long as false positive (i.e., incorrectly classify a link as LoS) probability is kept low.

In summary, for Case 1, the target UE can perform label-based model performance monitoring for its own model without assistance from other entities (e.g., LMF, PRU).

While the discussion above emphasized self-contained model performance monitoring by the target UE, the same principle can be applied if another entity (e.g., LMF) is willing to assist the target UE with label related information.

In general, the LoS identification can be provided by an entity different from the model inference entity to have independent information for model monitoring. Thus, for a model deployed in one node (i.e. gNB, LMF or the UE), the LoS state of a link can be signalled from another node (i.e. another gNB, UE). For example, a UE hosting the AI/ML model and measuring the DL PRS may also be configured to transmit SRS. Hence the LoS state of a given PRS resource may be known to gNB at a given time. To assist the model monitoring of the model hosted by the UE, the LMF could relay to the UE an assistance data, e.g., which PRS resource propagated via a LoS link and at what time.

As shown below, in legacy UE-based positioning methods, LMF can already inform the UE of LoS link state in assistance data, which helps the UE to calculate the location.

TS 37.355 v18.0.0 – NR-DL-PRS-ExpectedLOS-NLOS-Assistance The IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance is used by the location server to provide the expected likelihood of a LOS propagation path from a TRP to the target device, or for all DL-PRS Resources of the TRP to the target device. -- ASN1START NR-DL-PRS-ExpectedLOS-NLOS-Assistance-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerFreqLayer-r17 NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerFreqLayer-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerTRP-r17 NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerTRP-r17 ::= SEQUENCE { dl-PRS-ID-r17 INTEGER (0..255), nr-PhysCellID-r17 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r17 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r17 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON nr-los-nlos-indicator-r17 CHOICE { perTrp-r17 LOS-NLOS-Indicator-r17, perResource-r17 SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerResource-r17 }, ... } NR-DL-PRS-ExpectedLOS-NLOS-AssistancePerResource-r17 ::= SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF LOS-NLOS-Indicator-r17

Thus, similar LOS/NLOS assistance data can be signaled to assist with model monitoring. With the assistance data on LoS links, model output accuracy can be calculated for model performance monitoring of both direct and assisted AI/ML positioning.

Monitoring approach 2 (Label-based) for assisted AI/ML positioning

For assisted AI/ML positioning (Case 3a/2a), the gNB (Case 3a) or target UE (Case 2a) has an AI/ML model taking multiple measurement inputs to produce multiple model outputs such as the ToAs to multiple TRPs. The ToAs can then further processed by the LMF to produce an estimate of the UE position using, for example, conventional triangulation optimization.

By leveraging the associated triangulation optimization, the AI/ML model can be monitored without collecting new test samples. As shown in Figure 5 (b), the positioning algorithm takes the estimated time of arrivals into conventional triangulation-based error minimization framework to search and determine the UE position. It can be expected that the minimization outcome will have smaller residual losses when the AI/ML models are operating in the correct environment and are generating correct time of arrivals than when the models are applied to an environment different than the one used to train the models.

Therefore, the residual losses from conventional triangulation-based error minimization positioning algorithms

minpUEidpUE,pTRPi−c⋅ti

can be considered as an audit on the performance of the AI/ML model generating the ToA estimates.

In Figure 8, we provide the residual losses from conventional triangulation-based error minimization positioning algorithms. The AI/ML models for supplying the ToAs are trained with a dataset for the {60%, 6m, 2m} parameter.

- When the trained model is operating in the same environment of {60%, 6m, 2m}, the residual losses shown in blue line are below 0.77 with a probability of 99%.

- When the trained model is operating in the substantially different environment of {40%, 2m, 2m} where the model performs badly, the residual losses shown in orange line are above 0.77 with a probability of 99%.

- When the trained model is operating in a moderately different environment of {40%, 6m, 2m}, where the test environment has drifted from the training set environment, but the model is still performing well, the distribution of the residual losses shown in green remain quite similar to those for the {60%, 6m, 2m} environment (blue line).

From this analysis, it can be concluded that the residual losses from the conventional positioning algorithms can be used as a reliable metric to detect model/environment mismatch. For the example shown here, one could determine a threshold for flagging model refinement/re-training, for example, threshold = 1 considering both blue and green curves. If the positioning residual losses are above this threshold, there is a high chance that the environment has drifted too far from the training environment and the model will need to be replaced or adjusted.

Figure 8: Residual losses from conventional triangulation-based error minimization positioning algorithms. The ML model is trained in the {60%, 6m, 2m} environment and tested in three environments: {60%, 6m, 2m}, {40%, 6m, 2m} and {40%, 2m, 2m}.

Based on the investigation above, the intrinsic model monitoring for AI/ML assisted approaches is a valuable model monitoring method. The complexity, latency, and power consumption are negligible since the computation is already part of the conventional triangulation method.

In literature, the important model monitoring metrics include false alarm rate (FAR), missed detection rate (MDR), and detection delay. For the simplistic case evaluated in Figure 8, they can be estimated as follows. Assume the threshold for declare the model has drifted or not is set at residual loss =1. This corresponds to CDF=99.5% for the ‘no drift’ cases ({60%, 6m, 2m} and {40%, 2m, 2m}), and CDF=3% for the ‘drift’ case ({40%, 6m, 2m}).

The model monitoring algorithm can monitor m positioning requests to decide ‘drift’ or not. If for all m positioning requests, the residual loss is less than 1m, then ‘no drift’ is declared; otherwise, ‘drift’ is declared. In this case,

- FAR = 1−0.995m, the probability that the environment has not significantly changed (stayed at blue or green curve), but ‘drift’ is declared.

- MDR = 0.03m, the probability that the environment has changed significantly (i.e., changed to orange curve), but ‘no drift’ is declared.

The detection delay is the time to observe m positioning occasions. Plugging in m=2 shows that this simple method can achieve FAR=1% and MDR=0.09%.

Monitoring approach 3 (Label-free): ML model self-monitoring

Data augmentation is an important and in fact indispensable component of training high performance deep learning models with limited data. In addition, data augmentation may also enable the model to self-monitor without collecting new test samples nor ground truth UE positions. One model-based self-monitoring method is described below.

- Model training with data augmentation

  - Given ground truth samples xi, yi

  - Train the model with a mixture of

    - Unmodified samples xi, yi

    - Data augmentation modified samples xi', yi

- Self-model monitoring at operation

  - Given positioning request data xj

  - Obtain a first target estimate yj using unmodified data xj

  - Obtain a second target estimate yj' using data augmentation modified data xj'

  - Check the difference between the two target estimates, dj=‖yj−yj'‖.

We investigate the effectiveness of this self-monitoring approach using the smallest model (0.73 M parameters) for the centralized direct positioning approach trained with {60%, 6m, 2m} dataset samples. The CDF of the test results are summarized in Figure 9. It can be observed that:

- When the model is operating in environments similar to that it was trained for (such as the {60%, 6m, 2m} and {40%, 6m, 2m} environments), the two UE position estimates do not differ by more than 1 m, i.e., yj−yj'<1 m.

- When the model is operating in environments which it was not trained for and hence is under-performing, the two UE position estimates differ by more than 1 m with substantially high probabilities. For instance, in the {60%, 2m, 2m} and {40%, 2m, 2m} environments, the two UE position estimates differ by more than 1 m with 20% and 33% probabilities, respectively.

- While observing one UE position estimate difference of above 1 m is not reliable enough to ensure model is still operating in the intended environment, checking the dj distribution in a small window of 10s-20s model inferences can achieve monitoring accuracy >99%.

  - Observing 14 UE position estimate differences of above 1 m is enough to ensure an environment change from {60%, 6m, 2m} to {60%, 2m, 2m} is identified with a probability > 95%.

    - Observing 21 UE position estimate differences of above 1 m is enough to ensure an environment change from {60%, 6m, 2m} to {60%, 2m, 2m} is identified with a probability > 99%.

  - Observing 8 UE position estimate differences of above 1 m is enough to ensure an environment change from {60%, 6m, 2m} to {40%, 2m, 2m} is identified with a probability > 96%.

    - Observing 12 UE position estimate differences of above 1 m is enough to ensure an environment change from {60%, 6m, 2m} to {40%, 2m, 2m} is identified with a probability > 99%.

Note that the need of observing multiple instances is no different than any other model monitoring methods, since poor performance at a single instance cannot tell whether the model has truly degraded, or it’s a random anomaly.

Figure 9. 2D position estimate difference using unmodified or modified positioning request data at production in different operating environments for a small centralized direct positioning model trained with {60%, 6m, 2m} dataset samples.

The curves Figure 9 can be used to devise a monitoring method and calculate false alarm rate (FAR), missed detection rate (MDR), and detection delay as well. Take the threshold of 2D position estimate difference dthresh= 1m. At this point, CDF=99.96% for {60%, 6m, 2m} and {40%, 6m, 2m}, CDF=80% for {60%, 2m, 2m}, and CDF= 67% for {40%, 2m, 2m}. The model monitoring protocol can be:

- Declare ‘no drift’ from {60%, 6m, 2m} to {60%, 2m, 2m} if the last m positioning instances have d<dthresh.

- Declare ‘no drift’ from {60%, 6m, 2m} to {40%, 2m, 2m} if the last m position instances have d<dthresh.

- Otherwise, declare ‘drift’ and the model trained for {60%, 6m, 2m} is no longer appropriate.

In this case, take m=21,

- FAR = 1 – 0.999621 = 0.8%

- Missed detection rate of drift from {60%, 6m, 2m} to {60%, 2m, 2m}= 0.821 = 0.9%

- Missed detection rate of drift from {60%, 6m, 2m} to {40%, 2m, 2m}= 0.6721 = 0.02%

The detection delay is the time to observe m positioning occasions.

Thus, compared with the label-based model monitoring methods (e.g., see analysis in section 11.1.2), the self-model monitoring method requires longer observation time. On the other hand, it has the advantage of being self-contained and label free, and requiring no external assistance or information exchange beyond the model and its own data augmentation.