3GPP TSG RAN WG1 #120bis		R1-2501947

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.1.2

Source:	Google

Title:	ML based Positioning

Document for:	Discussion/Decision

Introduction

In RAN1 #120 meeting, the following agreements on ML based positioning were agreed.

Conclusion For AI/ML based positioning Case 3a, regarding the time stamp in a measurement report from gNB to LMF, - Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24} Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label. Agreement For AI/ML based positioning, - When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. - When label is to be reported/sent, time stamp of label is the time instance for which the label is valid. Agreement Send Reply LS R1-2501522 to SA2 about Case 2b with the content of Section 2 of R1-2501521 by revising “RAN1 would continue work and may provide additional input in the future to SA2 for Case 2b if any new agreement reached in RAN1.” To “RAN1 may provide additional input in the future to SA2 for Case 2b if any new agreement reached in RAN1.” Agreement Send Reply LS R1-2501524 to SA2 about Case 3b with the content of Section 3 of R1-2501521 by adding all the agreement made in this meeting for Case 3b. Agreement Final LS R1-2501523 is endorsed. Final LS R1-2501525 is endorsed. Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. Conclusion For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement. Agreement RAN1 send an LS to RAN3 and RAN2 to inform them above agreements and conclusions. Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above. Agreement Draft LS R1-2501627 is endorsed in principle. Agreement Final R1-2501628 is endorsed. Email approval by Feb 25

In this contribution, we provide some discussion on AI/ML based positioning.

Enhanced path-based measurement

In RAN1 #119 meeting, the following on path-based measurement enhancement has been agreed.

Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

The remaining issue is the path-based measurement by UE. One possible way is that the UE reports the path-based measurement results and the gNB selects the path. However, such operation could result in more overhead for the UE report. Therefore, one simple way is that the UE performs the path selection and the gNB transmits the UE report to the LMF. Then the enhanced path-based measurement can also be extended to the UE report.

In addition, there could be some measurement error for the path-based measurement. One possible way is to report the L1-SINR in addition to the path-based measurement. Then the NW can decide whether to perform additional processing for the path-based measurement or not. For example, the NW may perform some filtering to improve the accuracy or decides not to use the results for inference.

Proposal 1: Support to extend the enhanced path-based measurement to UE-side measurement

Proposal 2: Support to report the L1-SINR in addition to the path-based measurement with regard to potential measurement error for the path-based measurement.

Model monitoring

In RAN1 #117, RAN1 #118 and RAN1 #119 meeting, the following on model monitoring for Case 1 has been agreed.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

The first remaining issue is the monitoring outcome for Option A. One simple way is to report whether the performance failure or not to the NW. Thus, the UE can calculate the metric and compare the metric with a certain threshold, which may be configured by the NW. Then it can report the outcome of the comparison to the NW. The NW may decide to activate or deactivate the configuration for the AI/ML based positioning.

The second remaining issue is whether to support Option B (LMF performs the metric calculation) in addition to Option A. Since the Option B can achieve the same functionality as Option A, it is unnecessary to support a redundant option. Further Option B may result in potential risk for the UE to disclose its location.

Proposal 3: For Option A based model performance monitoring for Case 1, support a 1-bit indication on whether a performance failure is detected or not as the report content

- UE identifies the performance failure based on the calculated metric and threshold configured by the NW

Proposal 4: Do not support Option B based model performance monitoring for Case 1

Assistance information

In RAN1 #119 meeting, the following on assistance information has been agreed.

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

The first issue is whether the geographical coordinates of the TRPs (Info #7) should be provided by the NW. Without such information, it is hard for the UE to identify its absolute location. Instead, the UE may identify a relative location. Based on the Infor #7 and the predicted relative location, UE is able to identify the absolute location. Therefore, the Info #7 should be necessary. One possible way is to provide it based on associated ID, and the other possible way is to provide it explicitly. Since the location is a TRP-specific metric, if it is provided based on the associated ID, the dimension of the associated ID could get increased quite a lot. Therefore, a simple way is to provide it explicitly.

Proposal 5: Support Alt4 where Info #7 is provided explicitly from LMF to UE.

Conclusion

In this contribution, we provided discussion on AI/ML based positioning. Based on the discussion, the following proposals are provided.

Proposal 1: Support to extend the enhanced path-based measurement to UE-side measurement

Proposal 2: Support to report the L1-SINR in addition to the path-based measurement with regard to potential measurement error for the path-based measurement.

Proposal 3: For Option A based model performance monitoring for Case 1, support a 1-bit indication on whether a performance failure is detected or not as the report content

- UE identifies the performance failure based on the calculated metric and threshold configured by the NW

Proposal 4: Do not support Option B based model performance monitoring for Case 1

Proposal 5: Support Alt4 where Info #7 is provided explicitly from LMF to UE.