3GPP TSG-RAN WG3 Meeting #130	R3-258480

Dallas, US, 17th – 21st November, 2025

Title: 	Discussion on the Reply LS from SA2 on AI/ML Positioning Case 3a

Source: 	Huawei, Nokia, Qualcomm, Samsung

Agenda item:	9.2.11

Document Type:	Discussion and decision

1. Introduction

This contribution discusses the content of the incoming reply LS from SA2 in [1] on AI/ML Positioning Case 3a and the potential impact on RAN3 specifications.

2. Discussion

At this meeting we received a LS from SA2 in [1] – which is the reply LS to the LS sent by RAN3 in [2] where RAN3 asked SA2 to introduce support of related LMF procedures in TS 23.273 as needed, following the RAN3 decision to proceed with “Solution 1: Positioning Information collection for UEs under positioning” in Rel-19 – and below we report an excerpt of such LS from SA2:

# 1 Overall description SA2 thanks RAN3 for the reply LS on AI/ML Positioning Case 3a. SA2 would like to inform RAN3 that the related LMF procedure to support data collection for UEs under positioning by the NG-RAN has been added in clause 6.23 of TS 23.273. # 2. Actions To RAN3: ACTION: SA2 kindly asks RAN3 to take the above information into account.

In the reply LS, SA2 informs RAN3 that they introduced a new clause 6.23 in TS 23.273 where the LMF procedure to support data collection for the UEs under an ongoing positioning session (the so called “opportunistic data collection” in RAN3, or Solution 1 in [2]) is specified. Below an excerpt from TS 23.273 [3]:

## 6.23 Procedures of data collection for NG RAN-sided models for AI/ML positioning ### 6.23.1 General This clause describes the procedure for data collection by the NG RAN for input data to train or monitor the performance of the ML model for NG RAN-sided models. ### 6.23.2 Data Collection for NG RAN-sided models for UEs under positioning Figure 6.23.2-1 shows a procedure for data collection for NG RAN-sided models for UEs under positioning. Figure 6.23.2-1: Data Collection for NG RAN-sided models for UEs under positioning Precondition: There is an ongoing positioning session for the UE/PRU, and the network assisted or network based positioning is used. 1. The procedure in Figure 6.11.2-1 can be used for data collection by the NG RAN for input data to support NG-RAN node assisted positioning with gNB-side model. The steps 1-6 of network assisted positioning procedure are used as defined in clause 6.11.2 and the information to indicate that data collection is needed for the UE being positioned is included in the network positioning message in steps 5-6 as defined in NRPPa signalling in TS 38.455 [15]. 2. To provide the data for the UE being positioned, before LMF provides data to the NG-RAN, the LMF performs the user consent check by interaction with UDM as step 4 of data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements as defined in clause 6.22.3. If user consent is not granted, LMF sends to the NG-RAN an appropriate indication as defined in TS 38.455 [15]. 3-4. The LMF determines the data requested by NG-RAN based on the ground truth data of the UE. The LMF transfers a Network Positioning message to the NG-RAN node the requested data as defined in TS 38.455 [15]. If the LMF is unable to provide the positioning information, LMF sends to the NG-RAN an appropriate indication as defined in TS 38.455 [15].

In the above excerpt, we focus on the cases where data from LMF, i.e., ground-truth labels for the UE, cannot be provided to the gNB which previously requested such data to (re-)train or monitor the performance of the AI/ML model.

As specified by SA2, before providing data to the gNB, the LMF needs to check the user consent via interaction with the UDM. Assuming that the procedure for data collection as described in clause 6.23 of TS 23.273 is supported at LMF side (the assumption here is needed to exclude the cases where the LMF indicates to the gNB that it does not support the feature, by notifying the gNB with a “not supported” cause value in the NRPPa signalling), there could be the following cases:

- User consent is granted AND data is available at the LMF: the LMF provides the data to the gNB (first part of the description of “steps 3-4” in the procedure in clause 6.23);

- User consent is NOT granted EVEN IF data is available at the LMF: the LMF cannot provide the data to the gNB and informs the gNB with an appropriate indication as defined in NRPPa (second part of the description of “step 2” in the procedure in clause 6.23);

- User consent is granted BUT data is NOT available at the LMF: the LMF cannot provide the data to the gNB and informs the gNB with an appropriate indication as defined in NRPPa (second part of the description of “steps 3-4” in the procedure in clause 6.23);

- User consent is NOT granted AND data is NOT available at the LMF: this case is not captured in SA2 specification, but it should be easily acknowledged that the consequence is the same as for case b) and case c), that is, the LMF cannot provide the data to the gNB and informs the gNB with an appropriate indication as defined in NRPPa.

Based on the above the following Observations can be acknowledged:

Observation 1: Assuming that the LMF support the data collection feature, data from the LMF can be provided only if both conditions of “user consent granted” and “data available at LMF” hold.

Observation 2: Regardless of whether the motivation is “user consent not granted” or “data not available at LMF”, the consequence is exactly the same, that is, the gNB will not be provided with any data.

In the latest Rel-19 NRPPa, two codepoints in the Positioning Data Unavailable IE of the POSITIONING DATA COLLECTION REPORT message have been defined to make the gNB aware that data cannot be provided by the LMF, either because the feature is not supported at LMF side (codepoint “not supported”) or because data is not available at LMF side (codepoint “not available”), as follows [4]:

### 9.1.5 Messages for Positioning Data Collection Information Transfer Procedures #### 9.1.5.1 POSITIONING DATA COLLECTION REPORT This message is sent to report positioning information for data collection. Direction: LMF NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3; YES; reject. NRPPa Transaction ID; M; 9.2.4; -. LMF Measurement ID; M; INTEGER (1..65536, …); YES; reject. RAN Measurement ID; M; INTEGER (1..65536, …); YES; reject. Positioning Data Information; O; 9.2.107; YES; ignore. Positioning Data Unavailable; O; ENUMERATED(not supported, not available, …); YES; ignore. a

Observation 3: In current NRPPa specification, there is already signalling in place that allows the LMF to inform the gNB that either the data collection functionality is not supported at LMF or that the data is not available at LMF.

By specifically focusing on the “not available” codepoint, in our view such codepoint is general enough to cover all the possible cases previously described – case b), case c) and case d) above – that prevent the LMF to provide data to the gNB. The same applies also for the case of “not supported” codepoint, for which we see no reason to specify the exact reason for which the functionality is not supported.

Observation 4: The “not available” and “not supported” codepoints in current NRPPa signalling are general enough to cover all the possible cases that prevent the LMF to provide data to the gNB.

Hence, we think that there is no need to either enhance the existing codepoints or to introduce new ones for the purpose of covering each and every case that does not allow the LMF to provide data as requested by the gNB. Conversely, if RAN3 agrees to enhance existing codepoints or introduce new ones, this may create a significant burden in terms of future proofness, specification update/maintenance and potential misalignment among impacted specifications, e.g., TS 23.273 and TS 38.455 for the specific topic discussed in this paper. Moreover, any additional codepoints received in the signalling from LMF to gNB would not change the gNB behaviour, making such additional codepoints unnecessary.

Proposal: RAN3 to agree to use the existing codepoints in current NRPPa signalling to address all scenarios that prevent the LMF to provide data to the gNB.

3. Conclusion

Based on the discussion in this paper, we provide the following observations and proposal:

Observation 1: Assuming that the LMF support the data collection feature, data from the LMF can be provided only if both conditions of “user consent granted” and “data available at LMF” hold.

Observation 2: Regardless of whether the motivation is “user consent not granted” or “data not available at LMF”, the consequence is exactly the same, that is, the gNB will not be provided with any data.

Observation 3: In current NRPPa specification, there is already signalling in place that allows the LMF to inform the gNB that either the data collection functionality is not supported at LMF or that the data is not available at LMF.

Observation 4: The “not available” and “not supported” codepoints in current NRPPa signalling are general enough to cover all the possible cases that prevent the LMF to provide data to the gNB.

Proposal: RAN3 to agree to use the existing codepoints in current NRPPa signalling to address all scenarios that prevent the LMF to provide data to the gNB.

4. Reference

- R3-258004 (S2-2509721), Reply LS on AI/ML Positioning Case 3a, Rel-19, SA2 (CATT)

- R3-255935, Reply LS on AI/ML Positioning Case 3a, Rel-19, RAN3 (Nokia)

- 3GPP TS 23.273, 5G System (5GS) Location Services (LCS); Stage 2, Rel-19, V19.4.0 (2025-09)

- 3GPP TS 38.455, NG-RAN; NR Positioning Protocol A (NRPPa), Rel-19, V19.0.0 (2025-09)